Egerton University

December 2013

Instructions

Answer question one and  any other two

Question One (30 marks) Compulsory

- What is a logic gate?								[2 marks]

- With the aid of a diagram explain the operation of an S-R flip-flop. Explain how the unwanted condition of setting S=R=1 can be eliminated.			[5 marks]

- Explain how the following are represented in a digital system;

- Text

- Numbers

- Images

[6 marks]

- Design a combinational circuit that complements an entire 8-bit binary string whenever activated by a control signal. A HIGH on the control signal causes the circuit to complement the 8-bit string and a LOW does not. Explain the operation of the circuit works.

[7 marks]

- Carry out the following arithmetic operations;

- 3210 – 3710  using two’s complement

- 1410  *  1410 in binary

- 678 + 778  in binary

- 810 + 910     using  BCD

- 1616 + 4F16  in binary

[10 marks]

Question Two(20 marks)

- Explain two applications of multiplexers.					[2 marks]

- Differentiate between the following;

- Ripple and Asynchronous counters

- Digital and Analog signals

[4 marks]

- With the aid of a diagram explain the operation of a 4 to 1 multiplexer.													[4 marks]

- Explain any two applications of BCD in digital circuits.			[4 marks]

- With the aid of a diagram describe how you can read signals from sensing elements through a serial port of a CPU.						[6 marks]

Question Three (20 marks)

- Define a binary counter and state any two areas of application.		[3 marks]

- With the aid of a diagram and waveforms explain the operation of a mod-8 ripple counter.									[6 marks]

- Design and explain the operation of a combinational circuit that counts the number of people entering a shop. State the assumptions made if any.			[11 marks]

Question Four (20 marks)

- Explain why hexadecimal and binary number systems popular with digital systems.

[2 marks]

- Convert 236 to Gray code and state why the code is used for indicating shaft positions in rotating machines instead of pure binary.					[4 marks]

- With the aid of diagrams, explain how a logic pulser and logic probe are used in digital circuit trouble shooting.							[6 marks]

- A circuit with three inputs a, b, and c gives the following truth table with two independent outputs x and y. Implement the simplified expressions for x and y.

Inputs	       	Outputs

a	b	c  	x	y

0	0	0   	0	0

0	0	1	1	0

0	1	0	1	0

0	1	1	0	1

1	0	0	1	0

1	0	1	0	1

1	1	0	0	1

1	1	1	1	1

[8 marks]

Question Five (20 marks)

- What is the difference between combinational and sequential logic?	[2 marks]

- State the similarities and application of the following;

- Truth table

- Karnaugh Map

[4 marks]

- Explain any three reasons for digital circuit minimization.			[6 marks]

- Implement a simplified expression for X = A BC +  A B C  +  A B C   by using NAND gates only. Explain any two disadvantages of using NAND gates for implementing the expression.

[8 marks]