# KENYA AERONAUTICAL COLLEGE

DIPLOMA IN AERONAUTICAL ENGINEERING

YEAR 3, TERM 1

COMMUNICATION &NAVIGATION SYSTEMS

DIP 08 AVIONICS.

END TERM

NAME: __________________

DATE: November 16, 2012				TIME:  3 HOURS

STARTING TIME: 1300 HRS

Instructions:

- Attempt all the questions

- (a)	Define the following with respect to receivers.       		 (3 marks)

- Sensitivity

- Selectivity

- Image frequency

(b)	(i)	Draw a well labeled block diagram of a frequency modulated receiver and

explain the function of each block.

(ii)	Outline three advantages of envelope detector.          (3 marks)

(c)	 A super heterodyne receiver has an IF of 470KHZ and is tuned to 1065 KHZ. Given that C=0.1F and L is 1.2 mH. Determine:

- Local oscillator frequency

- Image frequency

- Image rejection ratio in dB             		   (6 marks)

- (a)	(i)	State two advantages of low level modulation

(ii)	With the aid of a well- labeled diagram, explain the operation of a varactor diode modulator.					(9 marks)

(b)	Show that the capacitance change introduced in a modulator is given by  							    (7 marks)

(c)	A double side band transmitter radiates 200Kw when modulated to a depth of 60%.Determine

- The depth of modulation necessary to increase the radiated power to 220 Kw

- The percentage of the lower side bands power to the new  radiated power(4)

- Discuss the following radio navigation aids using well labeled diagrams with emphasis on operation, frequencies, and components

- Instruments landing system

- Distance measuring equipment                			(20 marks)

- (a)	(i)	Outline two application of radar beacons

(ii)	Explain the operation of pulsed radar system using a block diagram

(10marks)

(b)	(i)	Show that Doppler frequency in a radar system is given by (4 marks)

(ii)	A fighter aircraft travels through space at a radial velocity of 1000km/hr with a transmission beam of frequency 10 GHZ.

Calculate the Doppler shift 						(2 marks)

(c)	A radar system operating at 6 GHZ radiate 600Kw towards an object 100km away. If the capture area is 8m2. Find the minimum receivable power. 	(4 marks)

- (a)	Define the following with respect to antennas

- Polarization

- Radiation resistance

- Beam width

(b)	Derive the expression for the field strength of a practical antenna such that

(5 marks)

(c)	(i)	Using the diagram below show that power radiated by a current element is given

by  from first principle			(8 marks)

(ii)	Outline four advantages of Yagi antenna   			  (4 marks)