# KENYA AERONAUTICAL COLLEGE

DIPLOMA IN AERONAUTICAL ENGINEERING

YEAR 3, TERM 1

GAS TURBINE ENGINES

DIP 08MECHANICAL.

END TERM

NAME: __________________

DATE: November 13, 2012				TIME:  2 ½ HOURS

STARTING TIME: 0800 HRS

Instructions:

- Attempt all the questions

- (a)	Outline TWO functions of each of the following aircraft engine parts

- Nozzle guide vanes

- Inlet guide vanes

- Turbines						(6 marks)

(b)	Explain the operation of each of the following types of aircraft engine turbines

- Impulse

- Free

- Reaction 	 					(6 marks)

(c)	With the aid of labeled sketches show FOUR methods of attaching turbine blades to the turbine discs of an aircraft engine			(8 marks)

- (a)	Describe the term fault free fire detection system as applied to aircraft engines

(2 marks)

(b)	Explain the conditions of aircraft fires under each of the following

- Detection

- Containment

- Extinguishing						(6 marks)

(c)	With the aid of a sketch describe a typical engine fire extinguishing system using the fire control handle in the cockpit				(12 marks)

- (a)	Outline five functions of aircraft engine bleed air		(5 marks)

(b)	With the aid of a sketch explain the ground generator cooling system

(7 marks)

(c)	Sketch a twin sprol by pass gas turbine engine section and show internal air cooling and sealing system					(8 marks)

- Using a pressure volume diagram and a cross- sectional sketch, explain the construction, operation and generation of thrust of a TWO-stage single entry centrifugal flow compressor aircraft turbo-propeller.