SOUTH EASTERN KENYA UNIVERSITY

UNIVERSITY EXAMINATIONS 2016/2017

FIRST SEMESTER EXAMINATION FOR THE DEGREE OF

BACHELOR SCIENCE IN ELECTRONICS

ELC 404: INTEGRATED CIRCUIT DESIGN

7TH

DECEMBER, 2016

TIME:10.30-12.30 P.M

INSTRUCTIONS TO CANDIDATES







This paper consists of FIVE questions.

Answer question ONE and any other TWO questions.

Question ONE carries 30 mark while the other TWO questions carry 20

marks each

QUESTION 1, 30 MARKS [COMPULSORY]

a) What is an Integrated circuit

b) Differentiate between direct and indirect band gap

[2 marks]

[4 marks]

c) State two reasons why GaAS is used in many applications such as microwaves and ICs

[2 marks]

d) What is a charged coupled device (CCD)

[2 mark]

e) A particular layer of MOS circuit has resistivity of 10 Ohms-cm. the section is 55 μm long,

5 μm wide and 1 μm thick. Calculate

i.

ii.

Sheet resistance

Resistance of the layer

[ 3 marks]

[ 2 marks]

f) State tree advantages of using static CMOS

g) Compare IC based on MOS and bipolar transistor technologies

h) Draw the circuitry representation of I2L inverter

SEKU/09-12/2016/2017

[3 marks]

[3 marks]

[2 marks]

Page 1

i) Describe three general functions of computer register

j) Draw a CMOS schematic diagram implementation of D-latch

QUESTION 2 [20 MARKS]

a) Explain the functions of the following registers

[ 3 marks]

[4 marks]

i.

ii.

iii.

iv.

Memory address register

Program counter

Accumulator register

Memory data register

[2 marks]

[2 marks]

[2 marks]

[2 marks]

b) Describe the two types of charged coupled devices

c) Draw the logic operations- NAND gate and its truth table

[8 marks]

[4 marks]

QUESTION 3, [20 MARKS]

a) State two main factors considered during design of any basic circuit [ 2 marks]

b) State two models used by IC designers to shrink the size of a circuit

c) Define the following terminologies used in ICs designs

[2 marks]

i.

ii.

iii.

iv.

Epitaxy

Wafer

Metallization

Bonding

[1 mark]

[1 mark]

[1 mark]

[ 1 mark]

d) Using a physical structure of p-Si substrate with a thin film layer of SiO2 describe

the process of photolithography

QUESTION 4, [20 MARKS]

a) State three advantages of using dynamic CMOS

b) Draw a CMOS inverter circuit and its transfer characteristics

[12 marks]

[3 marks]

[5 marks]

c) State and explain three circuit elements used in CMOS and radio frequency design

[6 marks]

d) Draw the physical structure of BiMOS npn transistor

SEKU/09-12/2016/2017

[6 marks]

Page 2

QUESTION 5, [20 MARKS]

a) The figure 1 below shows a DTL circuit model with high inputs. Use it to attempts

questions that follow. Given that β = 40, in the diagram 5K=5kῼ and 2.2K=2.2kῼ,

+5=+5V, 0.7=0.7V, 0.2=0.2V and 0.8=0.8V

Given that all the outputs are high and all the system is saturated. Find

i.

ii.

iii.

iv.

The voltage at point P

The current I1

The current I2

The current IB

[2 marks]

[ 1 marks]

[ 1 marks]

[2 marks]

v.

Calculate the maximum collector current when the transistor is saturated [2 marks]

vi.

vii.

The current I3

The maximum load current, Vin and Iin

[2 marks]

[3 marks]

b) Using inverters, and gates and OR gates implement a 5 X 8 X 4 (V*P*Z)

programmable logic array with following outcomes

z1 ab d e a b c de bc de

z 2 a c e

z 3 bc de c de bd

z 4 a c e ce

SEKU/09-12/2016/2017

[7 marks]

Page 3