SCHOOL OF ENGINEERING

DEPARTMENT OF ELECTRICAL/ELECTRONICS

CRAFT CERTIFICATE IN ELECTRICAL AND ELECTRONICS ENGINEERING

MODULE I

END OF SERIES EXAMINATIONS

AUGUST/SEPTEMBER 2017

ELECTRONICS TECHNOLOGY

TIME: 2 HOURS

INSTRUCTIONS TO CANDIDATES:

- Answer ANY FIVE questions.

- Any examination IRREGULARITY will lead to DISQUALIFICATION.

- Indicate your FULL ADMISSION NUMBER in each Answer Sheet used.

- Cell phones are NOT allowed in the examination room

- Ensure you have both SCHOOL ID and EXAM CARDS in the examination room

- Scientific calculators and mathematical tables may be used where appropriate.

- Do not write your name anywhere on the answer booklet

Answer any FIVE questions

QUESTION 1

- Perform the following operations:					(6 marks)

- 101101012 to decimal

- 3268 to binary

- 2AFH to octal by first converting to decimal

- With the aid of circuit and waveform diagrams, explain the operation of a full wave bridge rectifier 									( 8marks)

- The figure below shows implementation of a logic gate				(6marks)

Switch A

Switch B

Battery	lamp

- State the logic gate

- With aid of its truth table, describe its operation

QUESTION 2

- Draw the symbols of the following logic gates				(3marks)

- NAND gate

- EX-NOR gate

- OR gate

- Using karnaugh map, simplify the Boolean equation				(5marks)

F(ABCD)=ABC D+ABC D+ABCD+ABCD+ABCD+AB CD

- Distinguish between  Hartley oscillator and clap oscillator using diagrams 	(6marks)

- With the aid of a diagram explain the operation of  full wave voltage doubler as used in power supplies 								(6marks)

QUESTION 3

- Define the term flip-flop as used in electronic design circuits		(2mark)

- Differentiate between intrinsic and extrinsic semiconductors 		(4marks)

- Define the following terms  as related to semicondutors			(8marks)

- Doping

- Forward bias

- Insulator

- P-N junction

- Given that RS = 3kΩ, Vin  ranges from 40-70V, RL= 2kΩ and zener voltage, Vz = 10V,

- Calculate the currents through RS ,through the load and zener diode current Iz

- How will these values be affected if source voltage increases to 70V?										(6marks)

QUESTION 4

- Define the term transducer							(2mark)

- Describe the operation of inductive transducers	 				(6marks)

- The following bytes (shown in hexadecimal) represent a persons name as it would be stored in a computer’s memory. Each byte is a padded ASCII code. Determine the name of the person.

- 52 	41	4E	43	49	53				(3marks)

- Outline FOUR method of coupling amplifiers					(4marks)

- Using a diagram and waveforms, describe how a reservoir capacitor is used in smoothing dc output waveforms in power supplies						(5marks)

QUESTION 5

- (i) Define the term parity bit							(2mark)

(ii) State TWO types of parity bits						(2marks)

- Using  block diagrams  differentiate between current series feedback and current shunt feedback connections								(8marks)

- Outline THREE  advantages of Negative feedback as applied in Feedback amplifiers(3marks)

- Define the term rectification as used in power supplies			(2mark)

- The table below represents a truth table of a logic gate.				(3marks)

| A | B | A+B |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

- State the logic gate represented

- Show how two basic logic gates can be connected to form the gate in (I) above

QUESTION 6.

- using a diagram explain how a zener diode is applied in voltage regulation	 (5marks)

- State any THREE  logic families used in design and construction of logic gates.  (3marks)

- Explain the formation of N-type extrinsic semi-conductor material 		(5marks)

- Perform the following operations:						(7marks)

- 11012 + 110112

- Binary 11112 to gray code

- 11001÷101

*****END****