SCHOOL OF ENGINEERING

DEPARTMENT OF ELECTRICAL/ELECTRONICS

ARTISAN CERTIFICATE IN ELECTRICAL INSTALLATION

YEAR I

END OF SERIES EXAMINATIONS

AUGUST/SEPTEMBER 2017

ELECTRICAL INSTALLATION (TRADE THEORY)

TIME: 2 HOURS

INSTRUCTIONS TO CANDIDATES:

- Answer ALL questions in SECTION A  and SECTION B.

- Any examination IRREGULARITY will lead to DISQUALIFICATION.

- Indicate your FULL ADMISSION NUMBER in each Answer Sheet used.

- Cell phones are NOT allowed in the examination room

- Ensure you have both SCHOOL ID and EXAM CARDS in the examination room

- Scientific calculators and mathematical tables may be used where appropriate.

- Do not write your name anywhere on the answer booklet

SECTION A (40marks)

Answer ALL questions in this section

QUESTION 1

- Name the instruments used for measuring each of the following electrical quantities: 											(3marks)

- Voltage

- Power

- Energy

- Define the following quantities and give their SI units:			(4marks)

- Resistance

- Current

- State FOUR factors which affect resistance of a conductor			(4marks)

- Highlight FOUR ways of care and maintenance of secondary  cells		(4marks)

- A current of 5 A flows in the winding of an electric motor, the resistance of the winding being 100Ω. Determine

(i) the p.d. across the winding, and

(ii) the power dissipated by the coil.						(5marks)

QUESTION 2

- Outline the functions of the following components in an electric circuit	(4marks)

- Load

- Conducting wires

- A battery consisting of 4 cells in series each of emf 2.0V and internal resistance of 0.6Ω is used to pass a current through 1.6Ω resistor. Calculate the current through the battery. 										(4marks)

- Outline any FOUR types of capacitors 					(4marks)

- State Ohms law.								(2marks)

- Complete the table below showing the basic quantities, the SI units and symbols of the units. 									(6marks)

|  | Quantity | SI unit | Symbol of the unit |
| --- | --- | --- | --- |
|  | Length |  | M |
|  |  | second |  |
|  |  |  | A |
|  | Thermodynamic temperature |  | K |
|  |  | Kilogram |  |
|  | Amount of substance |  |  |
|  |  |  | Cd |

SECTION B (60 marks)

Answer ALL questions from this section

QUESTION 3

- Using a diagram, explain the construction of dry cells 				(8marks)

- State any two electric circuit conditions						(2marks)

- The following is a diagram showing resistors connected in a series.   		(5marks)

12V	12Ω	8Ω	6Ω

- Determine the total resistance of the circuit

- Calculate the current withdrawn from the power source

- Determine the current through 8Ω resistor.

QUESTION 4

- Differentiate between electromotive force and terminal voltage			(2marks)

- The figure below shows series parallel connection of resistors.calculate:

- The total resistance of the circuit

- The supply current

- Power dissipated by R4

- Energy consumed by R2 in 15 seconds					(13marks)

R1= 36Ω	R3=90 Ω

R2= 24Ω	R4=45 Ω

R5=120 Ω	R6= 9.6 Ω

V=144V

QUESTION 5

- Define the term capacitance as used in electrostatics				(2marks)

- State three factors which determine capacitance of a capacitor			(3marks)

- The figure below shows series parallel connection of capacitors. Determine:

- The total capacitance

- Total charge

- Voltage drop across C2							(10marks)

60μF

C1	C2		  20 μF

36V	C3	  30 μF	C4	18 μF

QUESTION 6

- Distinguish between local action and polarisation and explain how each shortcoming is reduced.					`				(4marks)

- Name TWO methods used to charge secondary cells				(2marks)

- State any two components of an electric cable					(2marks)

- A 12V battery is connected in a circuit having three series-connected resistors having resistances of 4Ω,9 Ω and 11 Ω. Determine; 					(3marks)

- the current flowing

- The p.d across the 9 Ω resistor

- Distinguish between primary cells and secondary cells giving two advantages of dry cells over secondary cells								(4marks)

******END******