ELECTRICAL INSTALLATION

QUESTION ONE

- Name three state authorities which are involved with power production and distribution.                                                                                                     (6marks)

- With an aid of labeled circuit diagram draw; A 3 phase 4- wire supply and show the distribution of both single and three phase loads                      (6mks)

- State two advantages of a.c system over d.c system                                   (2mks)

- State:

  - Two properties of each of the following conductor materials

- Copper

- Aluminium                                                                                                                         (4mks)

ii.       Two reasons why aluminium conductor is preferred in transmission of electricity                                                                                                                    (2mks)

QUESTION    TWO

- State:

- the difference between a joint and termination

- Three risks of loose terminations.                                                     (7mks)

- Draw a labeled diagram of a two-core PVC Armoured PVC insolated cable                     (6mks)

- Give the difference between crimping and cramping methods as used in joining    electrical conductors.                                                                       (4mks)

- Explain three tests conducted in completed electrical installation.(3mks)

QUESTION THREE:

- With an aid of a labeled diagram explain HRC fuse.                                     (7mks)

- State two advantages of circuit breakers over fuses.                                  (2mks)

- Explain three factors that influence the choice of a wiring system.          (6mks)

- Outline the procedure required to be used in order to save a person in contact with live conductor.                                                                             (5mks)

QUESTION FOUR

  - Define the following terms as used in protection.

- short circuit

- current rating (4mks)

  - state two advantages of using

- trunking over conduits

- ring over radial socket wiring	                                                             (4mks)

  - Outline:

- The purpose for earthing an electrical installation

- Three parts that need not to be earthed in an electrical installation.                   (6mks)

  - Explain with the aid of a diagram the current operated earth leakage circuit breaker.                                                                                                                (6mks)

QUESTION FIVE:

- Explain three factors that influence the choice of a wiring system.          (6mks)

- With an aid of a diagram explain three types of electrical join              (6mks)

- with aid of a labeled diagram, explain the following parts of an earthing system:

- earth electrode

- earth continuity conductor                                                                  (4mks)

- explain the following methods used in joining electrical conductors;

- cramping

- riveting                                                                                                    (4mks)