S O LA R INSTALLATION

QUESTION ONE

- Staten two:

- Advantages of using solar energy over other sources of electricity

- Factors to consider when choosing a wiring system for solar installation                                                                                          (4marks)

- Explain then need for:

- Sizing a solar installation system

- Servicing and maintaining a solar installation system.               (4marks)

- . define the following solar energy terminologies:

- Irradiance

- insolation                                                                                            (4marks)

- Explain four factors to be considered in the selection for location of a solar battery during installation.                                                                            (8marks)

QUESTION TWO

- describe the following accessories used in PV solar installation

- Ceiling roses

- Socket outlets                                                                                                  (4marks)

- Illustrate the connection of lighting circuits terminated at the following

- Joint box

- Three phase ceiling rose                                                                                (6marks)

- State any two;

- Methods of solar energy harvesting

- Application of solar energy                                                                           (4marks)

- A 12V d.c home solar system supplies a load which consists of four 10watt lamps and 80W television set. if the daily use for each appliance is 2 hours, determine the:

- total  daily energy demand;

- Daily system charge requirement.                                                               (6marks)

QUESTION THREE

- State one possible cause and remedy for the following solar installation

- Battery state is low

- No solar charge                                                                                               (8marks)

- With the aid of diagrams; show how solar energy is harvested using the following methods.

- Parabolic dish

- Flat plate collector                                                                                          (8marks)

- distinguish between the following wiring systems:

  - polyvinylchloride;

  - Polychlorepene.                                                                                (4marks)

QUESTION FOUR

- State two:

- Advantages of using solar water heater;

- Disadvantages of using solar cookers.                                           (4marks)

- outline how maintenance checks are carried out on the following parts of a  PV solar system:

- Inspecting collecting of dirty and cleaning of PV  solar module;

- Checking and topping up battery electrolyte.                              (8marks)

- Explain with an aid of a labeled block diagram the function of each component part of a photovoltaic system.                                                (8marks)

QUESTION FIVE

- State any two:

- Methods of solar energy harvesting;

- Applications of solar energy.                                                                        (4marks)

- Describe three factors considered during sizing of:

- Charge controller

- An inverter                                                                                                       (6marks)

- Describe with the aid of a labeled diagram the construction of flat plate collector.

(8marks)

- Explain the term 'depth of charge' as used in solar batteries.                             (2marks)