W1-2-60-1-6

JOMO KENYATTA UNIVERSITY

OF

AGRICULTURE AND TECHNOLOGY

UNIVERSITY EXAMINATIONS 2014/2015

……………………………………………. EXAMINATION FOR THE DEGREE

FOR BACHELOR OF SCIENCE IN ……………………………………………

SCH 2355: INSTRUMENTATION FOR ANALYTICAL CHEMISTS

DATE:  AUGUST, 2015                                                         TIME: 2 HOURS

INSTRUCTIONS:	ANSWER QUESTION ONE (COMPULSORY) AND ANY OTHER TWO QUESTIONS

QUESTION ONE (30 MARKS)

- i)	Give THREE (3) basic components of Analytical

instruments.

ii)	What is the function of analytical instruments.

iii)	Give FIVE (5) motivating factors for carrying out

measurements.								[7 marks]

- i)	Define a standard of measurement

ii)	What is calibration?

iii)	Give TWO requirements for a measurement to be useful.

[4 marks]

- i)	Explain the term ‘Doping’ as used in semi-conductor materials.

ii)	Give FOUR (4) characteristics of semi-conductor diodes.

iii)	TWO diodes, one of a germanium type and the other a silicon

types have equal rating.  If the silicon diode needs 0.60 volts

to conduct 10 MA, what voltage must be applied to a germanium

diode to get the same current flow.

[5 marks]

- i)	Explain the following terms:

- Signal-to-noise ration

- Sensitivity

- Detection limit

ii)	Give TWO (2) reasons why the amount of noise in an

instrument is important.

[6 marks]

- i)	Briefly describe a bipolar base current of a bipolar transistor.

ii)	GIVE THREE (3) components of the base current of

a bipolar transistor.

iii)	Define the short circuit current gain of a transistor.

[5 marks]

- i)	List TWO (2) reasons why linear integrated circuits need

additional 	components.

ii)	Give FOUR (4) advantages of using integrated circuits.		[3 marks]

QUESTION TWO (20 MARKS)

- i)	Define the a.c. resistance of a diode

ii)	Distinguish a signal diode from a power diode

[5 marks]

- i)	Plot a sketch of current flow versus bias voltage applied

to u-p semi conductor diode.

ii)	Explain briefly why a p-n junction has self-capacitance.

iii)	A silicon diode has a permittivity of 1.05 x 10-10 F/M2 is

reverse biased such that the depletion region has a width

of 4.5 x 10-6 M.  If the cross-section of the diode is a square

measuring 1.2mm on a side, what is the value of transition

capacitance.

[9 marks]

- i)	Give FOUR (4) desirable characteristics of an operational

amplifier.

ii)	Give the differences in amplification obtained when

an operational amplifier is operated with and without

negative feedback loop.

[6 marks]

QUESTION THREE (20 MARKS)

- Give the sources and methods of minimizing/elimination

of each of the following types of instrument noise.

i)	   Thermal noise

ii)	    Environmental noise

[11 marks]

- i)	List TWO (2) popular software signal enhancement technique

ii)	Show how each of the following hardware components are

used to discriminate signal from noise:

- Integrator circuit

- Lock-in or phase sensitive amplifier.

[9 marks]

QUESTION FOUR (20 MARKS)

- i)	A bipolar transistor is connected in the common base configuration.

Give TWO (2) reasons why the collector current is less than the

emitter current.

ii)	A n-p-n transistor connected in the configuration has an

emitter current of 3.2 mA and  a base current of 90A.

-	 calculate the collector current

-  use appropriate illustrations to show how

n-p-n transistor connected in the common

emitter configuration

[6 marks]

- i)	Give FOUR (4) advantages of using integrated circuits.

ii)	What is the load regulation of a rectifier circuit?

[4 marks]

- Briefly explain how full wave rectification of a.c. to d.c. voltage

may be achieved using two and four diodes.				[7 marks]

- How is a pH meter maintained to be in good working condition?	[3 marks]