SCHOOL OF ENGINEERING

DEPARTMENT OF ELECTRICAL/ELECTRONICS

CERTIFICATE IN ELECTRICAL AND ELECTRONICS ENGINEERING

MODULE I

END OF SERIES EXAMINATIONS

September/october 2017

ELECTRICAL PRINCIPLES

KNEC 1601/102

TIME: 2HOURS

INSTRUCTIONS TO CANDIDATES:

- Answer ALL questions.

- Any examination IRREGULARITY will lead to DISQUALIFICATION.

- Indicate your FULL ADMISSION NUMBER in each Answer Sheet used.

- Cell phones are NOT allowed in the examination room

- Ensure you have both SCHOOL ID and EXAM CARDS in the examination room

- Scientific calculators and mathematical tables may be used where appropriate.

- Do not write your name anywhere on the answer booklet

QUESTION ONE (20 MARKS)

- Define the following laws as used in d.c circuits.

- Kirchhoff’s  voltage law;

- Ohm’s law. 	                                              (4 marks)

- Determine the resistance of 1400m of copper having a diameter of 14mm  if the resistivity of copper is 1.7x10-8 Ωm                    (4marks)

- Figure 1 shows an electrical circuit network.

Determine the:

- Total resistance

- Voltage drop across 5 Ω resistor

- Power dissipated by 9 Ω resistor

- Energy consumed by 6 Ω resistor after 12min in joules

- Energy consumed by the whole circuit after 1hr 30mins in watt hours.   									 (12marks)

t				                        Figure 1

QUESTION TWO (20MARKS)

- Distinguish between;

- electrolysis and electroplating

- Primary and secondary cells.                              (6marks)

- With an aid of well labeled diagrams, explain the process of charging a battery using a solar panel, without using a regulator circuit.                      										(8marks)

- Draw a labeled diagram of a lechlanche dry cell.	                   (4marks)

- Explain the effect of temperature change in electrical conductors.      										(2marks)

QUESTION THREE (20MARKS)

- A capacitor consisting of two metal plates each area 50cm2 and spaced 0.2mm apart in air is connected across 150V. calculate the:

- Energy stored by the capacitor;

- Electric flux density;

- Potential gradient.						          (8marks)

- With  aid  of diagrams  explain the principle  operation  of a  moving – coil Galvanometer 	                                                     ( 12 marks)

QUESTION FOUR (20 MARKS)

- Define  the following terms as used in electrostatics :-

- Electric flux density;

- Relative permittivity.                                                     (4marks)

- A capacitance of 3μf, 6μf and 18μf are connected in series across a 340Vsupply. Determine the;

- Charge on each capacitor,

- Potential difference across 18μf capacitor.              (4marks)

- A multiplier of internal resistor 5Ω is used for a full deflection of 15 mA. if the  main current supplies  a current of 1.5 A, calculate  the shunt resistor  required  to make  it operational	                   (6marks)

- With the aid of a labeled diagram, derive an expression for equivalent capacitance of four capacitors connected in series.                 (6marks)

QUESTION FIVE	(20MARKS)

- Define the following trerms as used in magnetism:

- Magneto motive force;

- Reluctance.						          (4marks)

- A resistor of 20Ω, inductor of 0.3 H and a capacitor of 100 µF are connected across a 220v, 50 Hz mains. Determine  the following:

- Impedance

- Voltage across R,L & C

- Power factor ( p.f) 		                                            ( 10mark

- Describe an electric bell. 	                                              (6 marks)