MURANG’A UNIVERSITY OF TECHNOLOGY

SCHOOL OF ENGINEERING AND TECHNOLOGY

UNIVERSITY EXAMINATIONS FOR THE DEGREE OF SCIENCE IN:

BUILDING CONSTRUCTION AND MANAGEMENT

3RD  YEAR 1ST   SEMESTER 2015/2016 ACADEMIC YEAR

CENTRE: MAIN CAMPUS

___________________________________________________________________________

COURSE CODE:  TLM 3313

COURSE TITLE: STRUCTURES I

EXAM VENUE: CR               		STREAM:  BSc IN CONSTRUCTION

DATE: 26/4/16                             		EXAM SESSION: 2.00 – 4.00 PM

TIME:  2 HOURS

Instructions

- Answer Question 1 (compulsory) and ANY other two questions

- Candidates are advised not to write on question paper.

- Candidates must hand in their answer booklets to the invigilator while in the examination room

Question one

- Differentiate between design load and characteristic load.(2mks)

- Discuss how shear failure can arise in reinforced concrete members and how such failures can be avoided.(4mks)

- Explain the requirements of good detailing in reinforced concrete structures (4mks)

- A reinforced concrete beam which is 300 mm wide and 600 mm deep is required to span 6.0 m between the centres of supporting piers 300 mm wide . The beam carries dead and imposed loads of 25 kNm−1 and 19 kNm−1 respectively. Assuming fcu= 30 Nmm−2, fy= fyv= 500 Nmm−2and the exposure class is XC1, design the beam.(8mks)

- Differentiate between post-tensioning and pre-tensioning in prestresed concrete structures(4mks)

- List and discuss any five advantages of prestressed concrete (5mks)

- Explain the factors influencing deflection in prestresed concrete members (3mks)

Question two

- Differentiate between the ultimate and serviceability limit states.(2mks)

- A reinforced concrete floor subject to an imposed load of 6 kNm−2 spans between brick walls as shown below. Design the floor for exposure class XC1 assuming the following material strengths:

fcu= 35 Nmm−2

fy= 500 Nmm−2(10mks)

- Sketch the reinforcement details for the  designed floor (3mks)

Question three

- Explain how failure can occur in a concrete footing:(3mks)

- A 400 mm square column carries a dead load (Gk) of 1000 kN and imposed load (Qk) of 350 kN. The safe bearing capacity of the soil is 200 kNm−2. Design a square pad footing to resist the loads assuming the following material strengths:

fcu= 30 Nmm−2

fy= 500 Nmm−2(8mks)

- Sketch the details of the reinforcement for the designed footing. (4mks)

Question four

- Discuss the different types of prestress losses that can occur in prestressed concrete members

(4mks)

- A rectangular concrete beam of cross-section 30 cm deep and 20 cm wide is prestressed by means of 15 wires of 5mm diameter located 6.5 cm from the bottom of the beam and 3 wires

of diameter of 5mm,2.5 cm from the top.

Assuming the prestress in the steel as 840 N/mm2, calculate the stresses at the extreme fibers of the mid-span section when the beam is supporting its own weight over a span of 6m.(6mks)

- If a uniformly distributed live load of 6kN/m is imposed, evaluate the maximum working stress in concrete. The density of concrete is 24kN/m3.(4mks)