MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2014/2015

SECOND YEAR, FIRST SEMESTER EXAMINATION FOR DIPLOMA IN ELECTRICAL ENGINEERING

EEE 2202: ELECTRICAL MACHINES I

DATE: AUGUST, 2015						      TIME: 1½ HOURS

INSTRUCTIONS:  Answer question one and any other two questions

QUESTION ONE – 30 MARKS

- State three ways of improving commutation in D.C machines.			(3 Marks)

- With the aid of well-labelled diagram explain the construction of a d.c machine(7 Marks)

- Explain three energy losses of a generator.					(3 Marks)

- State two applications of d.c motors.						( 2 Marks)

- An 8 pole lap-wound armature  of a d.c generator has 1500 conductors and a flux per pole of 0.03wb. Determine the emf generated when it is running at 500 rev/min.	(4 Marks)

- With the aid of a diagram differentiate between a long shunt and short shunt wound dc machines.										(4 Marks)

- Explain three types of motor protection.						(3 Marks)

- Draw the following arrangements of dc machines;

- Short shunt								(2 Marks)

- Long shunt								(2 Marks)

QUESTION TWO (15 MARKS)

- A 10kw shunt generator having an amature circuit resistance of 0.75𝛺 and a field resistance of 125, generates a terminal voltage of 250v at full load. Determine the efficiency of the generator at full load, assuming the iron, friction and windage losses amount to 600w.

(7 Marks)

- A 250v series motor draws a current of 40A. The amature resistance is 0.15𝛺 and the field resistance is 0.5𝛺. Determine the maximum efficiency of the motor.		(8 Marks)

QUESTION THREE (15 MARKS)

- A 6pole lap wound motor is connected to a 250vdc supply. The amature has 500 conductors and a resistance of 1𝛺. The flux per pole is 20mwb. Calculate:

- The speed of the motor							(3 Marks)

- the torque developed when the amature current is 40A			(3 Marks)

- The shaft torque of a motor driving a 100V dc shunt-wound generator is 25Nm. The amature current of the generator is 16 A at this torque. If the shunt field regulator is adjusted so that the flux is reduced by 15%, the torque increases to 35Nm. Determine the amature current at this new value of torque.					(5 Marks)

- Sketch the following load characteristics of shunt-wound motor;

- torque/amature current						(2 Marks)

- Speed/ amature current						(2 Marks)

QUESTION FOUR (15 MARKS)

- With the aid of a diagram, describe the dc motor starter.			(5 Marks)

- Explain how the speed of the following motors is controlled;

- Shunt-wound dc motor						(4 Marks)

- Series-wound dc motors						(6 Marks)