MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

STAGE TWO EXAMINATION FOR

DIPLOMA IN INFORMATION TECHNOLOGY

CIT 2153:  DATA COMMUNICATION AND NETWORKS

DATE: AUGUST 2016						     	TIME: 1 ½ HOURS

INSTRUCTIONS:  Answer question one and any other two questions.

QUESTION ONE (30 MARKS)

- List the three modes of data communication  				(3 marks)

- Define the terms  								(3 marks)

- Data signal

- Protocol

- Baud rate

- A multinational organization intends to hold a meeting with all its board of directors situated in different parts of the world.  To achieve this, the managing director has to apply a networking communication technique to hold the meeting.

- Which technology should the managing director use to facilitate communication?

(1 mark)

- Name two devices that are to be used together with the computer for this to be achieved.  								(2 marks)

- Give two advantages of using this internet service  			(2 marks)

- Distinguish the following terms as used in networking

- Network server and network workstation  				(2 marks)

- Peer-to-peer network and client/server network  			(2 marks)

- Half duplex and full duplex transmissions  				(2 marks)

- Synchronous and asynchronous  					(2 marks)

- What is the significance of cladding in fiber optic cable  			(2 marks)

- Explain the concept of redundancy in error detection and correction  	(3 marks)

- Describe the importance of the following communication devices

- NIC card  								(1 mark)

- Bridge  								(1 mark)

- Give two reasons why wireless communication is becoming more common than the cabled network  								(2 marks)

- Name three layers in the OSI model which are network support layers  	(2 marks)

QUESTION TWO (20 MARKS)

- Describe the following categories of Twisted pair cable

- Cat 3 									(2 marks)

- Cat 4									(2 marks)

- Cat 5									(2 marks)

- Name four types of resources which can be shared in a network  		(3 marks)

- Give any two factors that affect the performance of a Local Area Network  (2 marks)

- Transmission impairment means that the signal at the beginning of a medium is not the same as the signal at the end of a medium.  List and explain the THREE causes of transmission impairment.  							(6 marks)

- List any three services provided by the application layer.  			(3 marks)

QUESTION THREE (20 MARKS)

- With an aid of a diagram, describe the bus topology  			(6 marks)

- Define the following terms  							(4 marks)

- Computer network

- Data communication

- Give three factors that determine whether a communication system is a Local Area Network or a Wide Area Network.  						(3 marks)

- Give three devices which can be used in a bus topology.  			(3 marks)

- Describe the importance of terminator as used in bus topology  		(2 marks)

- List any two layers of the Internet model and describe the duties of each layer  (2 marks)

QUESTION FOUR (20 MARKS)

- Explain the concept of distributed processing in networking.  		(3 marks)

- Distinguish between a Data Circuit-Terminating Equipment (DCE) and a Data Terminal Equipment (DTE)  								(4 marks)

- With an aid of a diagram, describe how light signal travels through a fiber optic cable.  											(5 marks)

- Why would an organization prefer to use a switch instead of a hub when setting up a network 									 (4 marks)

- List two transmission media used in wireless transmission.  			(2 marks)

- Determine the type of a network characterized by:				(2 marks)

- Connection between computers, printers and other resources using UTP cables.

- Over 300 computers connected to share resources in a city.

QUESTION FIVE (20 MARKS)

- Briefly describe the following layers as used in OSI model  			(6 marks)

- Physical Layer

- Network Layer

- Application Layer

- Give three problems of networking computers in an organization 		 (3 marks)

- Distinguish between

- Thinnet and Thicknet coaxial cables.  				(2 marks)

- Bluetooth technology and Infrared transmission  			(2 marks)

- Give three advantages of fiber-optic cable over satellite in data communication  (3 marks)

- Give any two examples of network operating system.  			(2 marks)

- Explain the line of sight principle in wireless communication  		(2 marks)