MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

THIRD YEAR, FIRST SEMESTER EXAMINATION FOR THE DIPLOMA IN CIVIL ENGINEERING

ECV 2301: FOUNDATION ENGINEERING.

DATE: AUGUST 2016                                                                                     TIME: 11/2 HOURS

INSTRUCTIONS: -	Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- Define the term foundation and state two assumptions made in the design of foundations.												(3 marks)

- State three conditions under which piles can be used.				(3 marks)

- Give two examples of piles based on:

- Method of transfer of loads							(2 marks)

- Method of construction of piles						(2 marks)

- (i) Using the table provided, interpolate the bearing capacity factors for  to two decimal places.									(3 marks)

- Use the values obtained in (i) above to calculate the ultimate bearing capacity of:

- A strip footing 1m wide

- A square footing 3m x 3m wide

- A circular footing 3m diameter

Given:

- Unit weight of soil =1.8t/m3

- Cohesion=2t/m2

- Depth of all footings=1.5m

- Angle of internal friction ,					(6 marks)

- State five causes of failure in foundations.						(5 marks)

- A 2.5m strip footing is founded at a depth of 1.9m below the ground level in a homogeneous bed of dense sand having the following properties:

.  Determine the ultimate, net ultimate, net safe and safe bearing capacity of the footing.  Given for :

Nc=61, Nq=43, Ny=48.  Assume a factor of safety of 3.0				(6 marks)

QUESTION TWO (15 MARKS)

- State five types of foundations							(5 marks)

- The depth to which foundations should be carried depends upon four principal factors.  State them.											(4 marks)

- A square footing of 2m x 2m size is subject to a gross vertical load of 180t.  The depth of foundation is 1m.  The foundation soil consists of a deposit of dense sand having a bulk density of 1.85t/m3 and an angle of internal friction of 360.  Determine the factor of safety against shear failure.  Given for : Nc=60, Nq=42, Ny=47.			(4 marks)

- State two conditions under which mat foundations can be used.			(2 marks)

QUESTION THREE (15 MARKS)

Design a rectangular combined footing for two columns as shown in the figure below.  Take allowable soil pressure as 10t/m2.  Also draw the maximum shear force and bending moment diagrams.

QUESTION FOUR (15 MARKS)

- What is another name for a mat foundation?						(1 mark)

- The plan of a mat foundation with 9 columns is shown in the figure below.  Assuming that the mat is rigid, determine the soil pressure distribution at A, B, C and D.  All columns are 0.5m x 0.5m.										(14 marks)