MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

SECOND YEAR FIRST SEMESTER EXAMINATION

FOR THE DEGREE OF

BACHELOR OF TECHNOLOGY IN MECHANICAL ENGINEERING

EMT 3255:  ELECTRONICS

DATE: APRIL 2016					   	                 TIME: 2 HOURS

INSTRUCTIONS: 	Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- Distinguish between intrinsic and extrinsic semiconductors.  		(2 marks)

- State two types of n-type impurities  						(2 marks)

- Explain the term ‘Zener effect’  						(2 marks)

- Define the term rectification 	 						(2 marks)

- Draw a common collector configuration circuit of an NPN transistor  	(3 marks)

- State three advantages of negative feedback  				(3 marks)

- Draw a truth table for a 2-input NOR logic gate.  				(4 marks)

- State any THREE types of Flip – Flops  					(3 marks)

- Define the term instruction set as used in microprocessors  			(2 marks)

- Convert (500)10 to binary  							(4 marks)

- State any THREE applications of microprocessors  				(3 marks)

QUESTION TWO (20 MARKS)

- With the aid of a circuit diagram and waveforms, explain the operation of a full wave rectifier.  									(10 marks)

- Sketch the input characteristics of a CE amplifier  				(3 marks)

- With the aid of waveforms, distinguish between class A and class B amplifiers  											(7 marks)

QUESTION THREE (20 MARKS)

- Perform the following number conversions

- (1  1  0  1  1  0)2 to decimal

- (1  1 0   0 1  1 )2 to decimal   					(6 marks)

- Draw the truth-table for the following logic circuit  				(8 marks)

DIAGRAM

- Define the term flip-flop  							(2 marks)

- Distinguish between half adder and full adder  				(4 marks)

QUESTION FOUR (20 MARKS)

- With the aid of a block diagram, describe the element of a microcomputer   (8 marks)

- State any THREE addressing modes used in 8085 Microprocessor.  	(3 marks)

- Write a simple assembly language program to add numbers 20 and 30 and store the result in register C.  								(9 marks)