MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

SECOND YEAR SECOND SEMESTER EXAMINATION

FOR DIPLOMA IN

CIVIL ENGINEERING

EMC 2250:  INTRODUCTION TO THERMODYNAMICS

DATE: APRIL 2016					   	                 TIME: 1 ½ HOURS

INSTRUCTIONS: 	Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- ..

- Name any four thermodynamic properties  				(2 marks)

- Describe the first law of Thermodynamics  				(3 marks)

- Differentiate between a closed and open system.  			(4 marks)

- ..

- Give the prefix and symbol for the following sub-multiples of basic units

- One million million

- One thousand million

- One millionth

- One thousand millionth.  						(2 marks)

- Define a reversible process and give three criteria for the Thermodynamic process to be reversible  						(5 marks)

- Define a Thermodynamic cycle  					(2 marks)

- What is specific Enthalpy and why is it considered to be a thermodynamic property. 								 (3 marks)

- Define the following Themodynamic processes.

- Adiabatic process 							(2 marks)

- Isochoric process 							(1 marks)

- Isothermal process   						(1 mark)

- Polytrophic process 						(2 marks)

- Isobaric process 							(1 mark)

- Isotropic process  							(2 marks)

QUESTION TWO (15 MARKS)

- State the second law of Thermodynamics  					(2 marks)

- Describe Throtthing  								(5 marks)

- Using the steady flow energy equation show shate for a throtthing process, the initial Enthalpy h1 is equal to the final enthalpy h2 					(8 marks)

QUESTION THREE (15 MARKS)

- Unit mass of a certain fluid is contained in a cylinder at a initial pressure of 20 bar.  The fluid is allowed to expand reversibly behind a piston according to the law PV2 = Constant until the volume is doubled.  The fluid is then cooled reversibly at a constant pressure until the piston regains its original position; heat is then supplied reversibly with the piston locked in position until the pressure rises to the original value of 20 bar.

Show this process on a Pv diagram.  						(5 marks)

- In a certain steam plant the turbine develops 1000 Kw.  The heat supplied to th steam in the boiler is 2800Kj/Kg, the heat rejected by the steam to the cooling water in the condenser Is 2100Kj/Kg and the feed pump work required to pump the condensate back into the boiler is 5Kw.

- Show a diagrammatic representation of the cycle  			(4 marks)

- Calculate the steam flow rate.  					(6 marks)

QUESTION FOUR (15 MARKS)

- Represent the following Thermodynamic process on Pv diagrams.

- Adiabatic process 							(4 marks)

- Polytropic process  							(4 marks)

- Using a diagrammatic illustration, derive the steady flow energy equation.

(7 marks)