MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

SECOND YEAR SECOND SEMESTER EXAMINATION

FOR DIPLOMA IN

CIVIL ENGINEERING

ECV 2250:  THEORY OF STRUCTURES III

DATE: APRIL 2016					   	                 TIME: 1 ½ HOURS

INSTRUCTIONS: 	Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- Explain the difference between a one way spanning slab and a two way spanning slab. 									(2 marks)

- Illustrate three types of reinforced beams by cross-section.  		(6 marks)

- A continuous one-away spanning slab has three equal spans of 3.5m each.  The slab depth is assumed to be 140mm.  The loading is as indicated below

Imposed loads = 3.0kNm-2

Dead loads = 5.2 kNm-2

Constants: 	 fcv = 30Nmm-2

Fy = 460 Nmm-2

Given that the exposure conditions are mild, and cover requirement is 25mm.

Design the slab and draw a sketch of the reinforcement distribution 	(8 marks)

- By quoting compressive and tensile strength of materials, explain why concrete requires reinforcement  							(4 marks)

- Give the purpose of these three elements of a simple structures  	(6 marks)

- Columns

- Beams

- Slabs

- State four factors which will affect the strength of reinforced concrete structures.

(4 marks)

QUESTION TWO (15 MARKS)

- By use of clear sketches with good lables, explain the three types of beams by cross-section.  									(6 marks)

- A reinforced concrete beam has an effective span of 9m, and carries a uniformly distributed dead and imposed loads as shown below

Qk=5kNm-1			fcv = 30Nmm-2

Gk = 4kNm-1			fy =500Nmm-2

Assuming a cross section of 200mm width by a depth of 370mm, design the bending reinforcement for the beam.  								(7 marks)

- State two factors which affect the durability of concrete structures.  	(2 marks)

QUESTION THREE (15 MARKS)

- State and explain at least two end conditions of columns for the purpose of design 										 (4 marks)

- Give three types of columns in terms of their dimensions and support.  	(3 marks)

- Determine suitable dimensions for the effective depth and width of the beam, with the loading shown below.  The effective span of the bema is 9m.

Qk = 25kN/m

Gk = 30kN/m

QUESTION FOUR (15 MARKS)

- Calculate the self weight of a reinforced concrete beam of breadth 300mm and depth 650mm and length of 6m.  Assume density of concrete to be 2400 kg/m3   (4 marks)

- Differentiate between the design strength and the characteristic strength of materials as used in design  								(2 marks)

- A short braced column carries load as shown below

Gk = 1000KN			fy =500Nmm-2

Qk = 1000 KN			fcv = 40Nmm-2

Design the longitudinal and links for the column given that the cross-sections 350mm square.  									(9 marks)