MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

FIRST YEAR, SECOND SEMESTER EXAMINATION FOR THE DIPLOMA IN ELECTRICAL ENGINEERING

ECV 2150: ELECTRICAL ENGINEERING PRINCIPLES II.

DATE: AUGUST 2016                                                                                     TIME: 11/2 HOURS

INSTRUCTIONS: -	Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- State three factors that determine the amount of voltage generated in the process of a.c generation										(3 marks)

- Define the following a.c terms							(5 marks)

- Instantaneous value

- Amplitude

- Frequency

- Q-factor

- Resonance

- Describe the effect of frequency on inductive reactance and current magnitude.	(5 marks)

- Derive an impression for resonant frequency.					(5 marks)

- With the aid of diagrams, illustrate the connection of three-phase, four wire star connected system.										(5 marks)

- Draw the wave form diagrams of a balanced three phase system for star or delta circuit												(3 marks)

- Explain two advantages of three phase over single phase circuits.			(2 marks)

QUESTION TWO (15 MARKS)

- A three branch parallel a.c circuit has the following in each branch:

Branch 1: 10A at an angle of 00

Branch 2: 30A at an angle of 400

Branch 3: 20A at an angle of -200

Calculate:

- (I)  Total current drawn from the supply

(II)  Phase angle of the supply current

- Represent your answer in the form

- Draw the phasor diagram showing three branch currents and the supply current.											(12 marks)

- With the aid of diagrams, show three different types of alternating waveforms.	(3 marks)

QUESTION THREE (15 MARKS)

- Show that the phase relationship between a voltage and current in a.c circuits is given by								(6 marks)

- A series R-L circuit containing a 200 resistor and a 0.5H inductor is energized by a 250v, 50Hz supply.  Calculate:

- The reactions of the inductor

- The impedance of the circuit

- The current the circuit

- The phase angle between the current and voltage

- The p.d across the resistor and the inductor

- Draw the phasor diagram of the circuit.					(9 marks)

QUESTION FOUR (15 MARKS)

- Explain the effect of incorrect connection of one phase in a delta connected system.												(4 marks)

- Show how loads can be connected to a generator using delta connection.		(5 marks)

- Three 10 resistors are connected to a balanced 440v three phase supply, if the resistors are connected in

- Star

- Delta

Calculate in each case, the phase current and the line current.		(6 marks)