MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.mucst.ac.ke  Email: info@mucst.ac.ke

University Examinations 2014/2015

SECOND YEAR, FIRST SEMESTER EXAMINATION FOR DIPLOMA IN ELECTRICAL ENGINEERING

EEE 0221: ELECTRICAL ENGINEERING PRINCIPLE III

DATE: DECEMBER 2014                                                                                        TIME: 112 HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- Define the term utilisation factor as applied to illumination			(1 mark)

- Differentiate between light and illumination					(1 mark)

- State Lambert’s Cosine Law of illumination					(1 mark)

- A lamp giving out 1500 lumens is suspended 3m above the horizontal plane.  Calculate the illumination at a point on the horizontal plane

- Directly below the lamp						(3 marks)

- 6m away from the vertical axis					(3 marks)

- State three properties of a well designed lighting scheme			(3 marks)

- The illumination in a drawing office 30mx10m is to have a value of 300 lux and is to be provided by a number of 500-W filaments lamps.  If the coefficient of utilisation is 0.5 and the depreciation factor 0.9.  Determine the number of lamps required.  The lamp efficiency of each lamp is 15 lm/w

(4 marks)

- Explain briefly four energy losses in a transformed				(4 marks)

- A single phase transformer has 500 primary and 1200 secondary turns.  The net cross-section area of the core is 50cm2.  If the primary winding is connected to a 50HZ supply at 450V, calculate:

- The peak value of the flux in the core					(2 marks)

- The voltage induced in the secondary windings			(2 marks)

- A three phase transformer has 900 primary turns and 150 secondary turns.  If the supply voltage is 2.5 kv, determine the secondary line voltage on no-load when the windings are connected

- Delta-Star								(3 marks)

- Star-Delta								(3 marks)

QUESTION TWO (15 MARKS)

A 500KVA transformer has a full load copper loss of 2.5 KW and an iron loss of 1.2 KW.  Determine the transformer efficiency at

- Full load and 0.85 power factor						(5 marks)

- Half load and 0.85 pf							(6 marks)

- Determine the output VA at which the efficiency of the transformer is maximum											(4 marks)

QUESTION THREE (15 MARKS)

- State four factors which affect the coefficient of ultilisation 			(4 marks)

- Explain briefly five types of lighting schemes				(5 marks)

- Design a suitable lighting scheme for a factory 100mx40m with a height of 7m.  Illumination required is 100 lux.  State the number of 50w fluorescent tubes giving 60lm/w.  (Use twin-tube fittings given that depreciation factor =1.2, utilisation factor=0.5, space height ratio of unity and mounting height =5m).		(6 marks)

QUESTION FOUR (15 MARKS)

- A 2400/415V single phase transformer takes a no=load current of 0.8A and the core loss is 400W.  Determine

- The value of working current						(2 marks)

- Power factor of the transformer					(2 marks)

- The value of magnetising current					(2 marks)

- Draw the no-load phasor diagram for the transformer		(1 mark)

- A single phase transformer has 2500 turns on the primary and 800 turns on the secondary.  It’s no load current is 4A at a power factor of 0.2 lagging.  Assume the volt drop in the winding is negligible, determine the primary current and power factor when the secondary current is 150A at a power factor of 0.85 lagging			(8 marks)