MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.mucst.ac.ke  Email: info@mucst.ac.ke

University Examinations 2014/2015

SECOND YEAR, SECOND SEMESTER EXAMINATION FOR DIPLOMA IN ELECTRICAL ENGINEERING

EEE 0230: ELECTRICAL INSTALLATION TECHNOLOGY IV

DATE: DECEMBER 2014                                                                                            TIME: 112 HOURS

INSTRUCTIONS: Answer questions one and any other two questions from section B

SECTION A

QUESTION ONE (30 MARKS)

- Outline the purpose of earthing						(3 marks)

- Define the following terms as used in relation to earthing

- Earth leakage								(2 marks)

- Earth-loop impedance							(2 marks)

- Earth has many failings as a conductor.  Explain				(4 marks)

- State two possible causes of the following electrical fault

- Electric shock								(2 marks)

- Fire/burning								(2 marks)

- Complete loss of supply						(2 marks)

- An electric heater is fitted to a copper kettle of mass 0.5 kg containing 1.5 litres of water.  Determine the amount of heat needed to raise the temperature of the water 200c to boiling point										(3 marks)

- Explain the classification of hazards areas					(6 marks)

- State the procedures to observe whenever an electrical fault occurs		(4 marks)

SECTION B

QUESTION TWO (15 MARKS)

- Define commission								(2 marks)

- Explain briefly the purpose of issuing a test certificate of a completed installation or additional of installation work.						(5 marks)

- State how negligence, misuse of abuse by the installer or user may result in electrical faults										(3 marks)

- With aid of a circuit diagram, describe high voltage direct current testing of cables. 											(5 marks)

QUESTION THREE (15 MARKS)

- Outline the purpose of protective multiple earthing				(3 marks)

- With aid of diagrams, explain the procedures of carrying out soil resistivity tests

(7 marks)

- Explain briefly why the earth electrodes must be situated outside the resistance area of any other electrode								(3 marks)

- State two method used for prevention of earth leakage			(2 marks)

QUESTION FOUR (15 MARKS)

- Explain briefly the following conditions which limit the choice of electrical machines

- Supply available							(2 marks)

- Rating									(2 marks)

- Describe the following types of motor enclosures

- Pipe-ventilated							(2 marks)

- Drip-proof								(2 marks)

- Petrol service pumps is a hazardous area, explain briefly the conditions required for its constructions and in station							(7 marks)