MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.mucst.ac.ke  Email: info@mucst.ac.ke

University Examinations 2014/2015

SECOND YEAR, SECOND SEMESTER EXAMINATION FOR DIPLOMA IN ELECTRICAL ENGINEERING

EEE 0232: ELECTRICAL MACHINES II

DATE: DECEMBER 2014                                                                                            TIME: 112 HOURS

INSTRUCTIONS: Answer questions one and any other two questions

QUESTION ONE (30 MARKS)

- Define 										(4 marks)

- Slip

- Frequency of motor current

- Draw the power flow diagram for induction motor				(5 marks)

- Give reasons why								(8 marks)

- An inductions machine must run at synchronous speed

- An induction machine can operate as an induction generator

- A four pole induction motor has a slip of 0.02.  Calculate			(9 marks)

- Synchronous speed

- Speed of motor

- Frequency of motor currents

- Draw a well labelled diagram of a presentation of a circle diagram		(4 marks)

QUESTION TWO

- Describe the difference in operational and constructional features of induction motor.  Use appropriate diagrams							(10 marks)

- State five reasons why induction motors are preferred for industrial applications

(5 marks)

QUESTION THREE

Show that:

- Xr=R for maximum torque						( 4 marks)

- Nr=s(1-Ns)								(3 marks)

- Fr=sf									(3 marks)

Explain the production of torque in induction motors.  Use appropriate diagrams	(5 marks)

QUESTION FOUR

A 415v 3Φ 50 Hz 4 pole star connected induction motor runs at 24 rev/s on full load.  If the motor resistance =0.35Ω and reactance = 3.5 Ω and motor-stator turns ratio is 0.85:1.  Calculate												(15 marks)

- Synchronous speed

- Slip

- Full load torque

- Power output if mechanical losses=700w

- The starting torque