MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.mucst.ac.ke  Email: info@mucst.ac.ke

University Examinations 2014/2015

SECOND YEAR, FIRST SEMESTER EXAMINATION FOR THE DEGREE OF BACHELOR OF SCIENCE (GENERAL) PHYSICS OPTION

SPH 2201: ELECTRICITY AND MAGNETISM II

DATE: DECEMBER 2014                                                                                            TIME: 2 HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- State Lenz’s law							(2 marks)

- Find the amount of energy stored in a 5uf capacitor when it is connected across a 120 V line									(3 marks)

- Differentiate between absolute and secondary instruments		(4 marks)

- State three applications of oscillations					(3 marks)

- Calculate the magnetic flux though each turn of a 300-turn, 7.2 mH coil when the current in the coil is 10mA.							(4 marks)

- Show that the energy stored in a capacitor is given by 	(5 marks)

- An alternating current is given by i=120sin 120πt.  Find:

- Maximum value						(1 mark)

- Frequency							(2 marks)

- Time period 							(2 marks)

- State four conditions which must be satisfied in order to apply Gauss’s law on a surface.										(4 marks)

QUESTION TWO (20 MARKS)

- At t=0, a 40MH inductor is placed in series with a resistance R=3.0 Ω and a charged capacitor C=4.8 uf.

- Show that this circuit will oscillate					(5 marks)

- Determine the frequency						(4 marks)

- What are the advantages of permanent magnet moving coil instruments	(7 marks)

- Calculate the inductance of an inductor if has a stored energy of 1.5 J when there is a current of 2.5 A in it.								(4 marks)

QUESTION THREE (20 MARKS)

- Show that the differential form of Gauss’s law is given by

where  is the volume charge density.					(6 marks)

- A parallel-plate capacitor has an area of 2 cm2 and plate separation of 2mm.  How much charge does this capacitor store when connected to a 6 V battery?(5 marks)

- The switch in a series RL circuit in which R=6Ω, L=3H and E=24 V is closed at t=0.  Calculate

- Maximum current in the circuit				(1 mark)

- Current when ==0.5 sec.					(4 marks)

- Distinguish between self and mutual inductance				(4 marks)

QUESTION FOUR (20 MARKS)

- A coil is placed near an electromagnet as shown below:

Find the direction of induced current in the coil

- At the instant the switch is closed						(2 marks)

- After the switch has been closed for several seconds			(2 marks)

- When the switch is open							(2 marks)

- A coil has an inductance of 3mH and a current through it changes from 0.2A to 1.5A in a time of 0.2 sec.  Find the magnitude of the average induced emf in the coil during this time										(5 marks)

- A moving coil instrument has a resistance of 10 Ω and gives full-scale deflection when carrying a current of 50 mA.  Show that it can be adopted to measure voltage up to 750 V and current up to 100 A							(7 marks)

- State two types of moving coil instruments					(2 marks)