MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.mucst.ac.ke  Email: info@mucst.ac.ke

University Examinations 2014/2015

SECOND YEAR, FIRST SEMESTER EXAMINATION FOR DIPLOMA IN ELECTRICAL ENGINEERING

EEE 0225: ELECTRONICS II

DATE: DECEMBER 2014                                                                                            TIME: 112 HOURS

INSTRUCTIONS: Answer questions one and any other two questions

SECTION A

QUESTION ONE (30 MARKS)

- List two types of regulations							(2 marks)

- Define rectification								(2 marks)

- The voltage gain of amplifier without feedback is 400.  If the feedback ration () required equal to 0.1, find the voltage gain of the amplifier with negative feedback	(3 marks)

- List four features of i.c voltage regulators					(4 marks)

- A voltage regulator has a load regulation of 20μv/mA.  Under no load conditions, the output voltage is 5v.  Calculate the voltage under full-load condition if the load current is 40 mA										(3 marks)

- State the requirement of feedback network					(3 marks)

- Differentiate between positive and negative feedback amplifier and give one application for each									(4 marks)

- Define OP-AMP and draw its symbol 					(3 marks)

- Draw the circuit of differentiator						(3 marks)

- Outline two classes of power amplifier					(3 marks)

SECTION B

QUESTION TWO (15 MARKS)

- List two types of rectifier							(2 marks)

- With the help of circuit and waveforms describe the operation of a single phase full wave rectifier																					(8 marks)

- Show that the output voltage of a half wave rectifier is VO=0.318VM	(5 marks)

QUESTION THREE (15 MARKS)

- Draw block diagram of power supply						(4 marks)

- The zener diode has the following ratings

VZ=8.8V at 1z=50mA

r2=2Ω		1z=50mA

1z (minimum) =4mA	1z (maximum) =120mA

- Calculate voltage regulation of the regulator				(5 marks)

- Calculate load voltage when current varies from 29mA to 110 mA	(6 marks)

QUESTION FOUR (15 MARKS)

- With aid of graphs show that negative feedback increases bandwidth	(4 marks)

- Define the following OP-AMP parameters

- Slew rate								(2 marks)

- Gain									(2 marks)

- Calculate the common-mode rejection ration (CMRR) of an op-Amp that has a differential gain of 200,000 and a common mode gain of 9.33		(3 marks)

- Differentiate inverting and non-inverting mode of an op-Amp		(4 marks)