MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.mucst.ac.ke  Email: info@mucst.ac.ke

University Examinations 2014/2015

FIRST YEAR, FIRST SEMESTER EXAMINATION FOR DIPLOMA IN ELECTRICAL ENGINEERING

EMC 0203: MATERIALS AND METALLURGY I

DATE: DECEMBER 2014                                                                                       TIME: 112 HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- Define each of the following terms as applied in materials science and engineering											(5 marks)

- Unit cell

- Phase diagram

- Alloy

- Homologous temperature

- Hardness

- Give  at least two specific engineering applications for each of the following materials

(5 marks)

- Dielectric

- Intrinsic semi-conductor

- Ferromagnet

- Elastomer

- Solder

- Sketch a well-labelled graph of electrical resistivity against absolute temperature for each of the following electrical materials						(5 marks)

- Normal conductor

- Super conductor

- Write equation for ohm’s law.  Define the symbols and give thri SI units	(3 marks)

- A wire whose diameter is 0.20 cm must carry a 20A current.  The maximum power dissipation along the wire is 4W/m (watts per meter).  Calculate the minimum allowable conductivity of the wire in (ohm meters)-1 for this application		(12 marks)

QUESTION TWO (15 MARKS)

- Distinguish the following materials						(712 marks)

- Hypo-entectoid steel and Hyper-entectoid steel

- Insulator and dielectric

- Alloy and compound

- Substitutional solution and interstitial solution

- Hard magnet and soft magnet

- A steel wire of diameter 5mm and length 10m carries a load of 3.40N at its end.  If the modulus of elasticity of steel is 200GPa and coefficient of thermal expansion for steel is 12x10-6/0c, Calculate:								(712 marks)

- The increase in length of the wire caused by the load

- The temperature rise that would cause the same increase in length as in b (i) above.

QUESTION THREE (15 MARKS)

- Define each of the following properties of materials				(6 marks)

- Resistivity

- Magnetostricion

- Hardenability

- Allotropism

- Grain boundary

- Invar effect

- With aid of neat sketches of ‘hard sphere models’ illustrate each of the following crystalline structure of metals							(9 marks)

- Body centred cubic

- Face centred cubic

- Hexagonal close packed

QUESTION FOUR (15 MARKS)

- Plot a well-labelled cooling curve for pure iron at a pressure of 1 atmospher

(6 marks)

- What are conditions for formation of each of the following types of alloys	(6 marks)

- Substitutional solid solution

- Interstitical solid solution

- Outline the three stages of annealing process					(3 marks)