MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2016/2017

THIRD YEAR, FIRST SEMESTER EXAMINATION FOR THE DEGREE OF BACHELOR OF TECHNOLOGY IN ELECTRICAL ENGINEERING.

EET 3300: DIGITAL ELECTRONICS

DATE: DECEMBER, 2016                                                                          TIME: 2 HOURS

INSTRUCTIONS: -	Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- Define the following

- Logic gate 								(1 mark)

- Flip-flops 								(1 mark)

- Sequential logic circuit						 (1 mark)

- Truth table								 (1 mark)

- Combination logic circuits						 (2 marks)

- Use the truth table to prove that 			(5 marks)

- by use of distributive law, express the expression below in standard product of sums form

(4 marks)

- develop a truth table for the S-R flip-flop shown below			(4 marks)

- (i) Define a multiplier 							(1 mark)

(ii) Implement a 2 input multiplexer using logic gates 			(4 marks)

- Define analog-to-digital conversion and give an example of an analog-signal (2 marks)

- Define

- RAM									 (2 marks)

- ROM									 (2 marks)

QUESTION TWO (20 MARKS)

- (i) State the advantages of utilizing JK flip-flops in sequential logic circuits. (2 marks)

(ii) Using the JK flip-flop for the figure below, develop its truth table, characteristic table and its excitation table

Fig 2.1 JK flip flop 								(12 marks)

- Realize a D flip flop using the SR flip flop 					(6 marks)

QUESTION THREE (20 MARKS)

- (i) Write down the three basic operations of Boolean algebra		 (3 marks)

- Develop a truth table for an AND operation with an AND gate of two inputs. 										(3 marks)

- Design a full adder with its truth table and develop its circuit implementation (14 marks)

QUESTION FOUR (20 MARKS)

- Design a JK flip-flop based circuit whose state diagram is shown below. 	(10 marks)

- Draw a basic microcomputer with its components				 (8 marks)

- Define a microcomputer							 (2 marks)