MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2016/2017

THIRD YEAR, FIRST SEMESTER EXAMINATION FOR THE DEGREE OF BACHELOR OF TECHNOLOGY IN MECHANICAL ENGINEERING.

EMT 3300: INTRODUCTION TO MICROPROCESSOR

DATE: DECEMBER, 2016                                                                          TIME: 2 HOURS

INSTRUCTIONS: -	Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- With the aid of a block diagram, explain the functions of the following microcomputer sections

- Microprocessor

- ROM

- Bus system

- I/O ports 									(10 marks)

- Explain the functions of the following microprocessor registers

- Program counter

- Instruction register

- Stackpointer 									(6 marks)

- State any two equipment used in microprocessors fault diagnosis 			(2 marks)

- An 8-bit microcomputer has 32 address lines.  Determine:

- The number of addressable memory locations

- The word size

- The memory capacity 								(6 marks)

- Write an assembly language program to add the contents of two memory locations FAOOH and FA01H and store the result in a third memory location FA02H.			(6 marks)

QUESTION TWO (20 MARKS)

- Define the following terms as used in memories

- Volatile

- Access time				 					(4 marks)

- A particular 8 bit microprocessor based system requires 4K bytes of RAM.  Available RAM chips are 1K x4 bits

- Determine the number of 1K x 4 bits chips required

- Show how the 1K x 4 bits chips should be connected to achieve the 4K bytes of RAM required for the system.							(13 marks)

- State any three advantages of semiconductor memories over magnetic memories 	(3 marks)

QUESTION THREE (20 MARKS)

- State two advantages and two disadvantages of the following I/O techniques

- Isolated

- Memory-mapped								(8 marks)

- Explain any three function of interfaces						(6 marks)

- State the three methods of parameter passing used in subroutines			(3 marks)

- State the three modes of DMA data transfer 						(3 marks)

QUESTION FOUR (20 MARKS)

- Study the program below and determine;						(12 marks)

- The number of bytes being added

- The address of the memory location where the final results is stored

- The length of the program in bytes

STA 0091H ORG 1800H

LXI 1800H

MOV A,M

MVI C,0AH

AGAIN	INX H

ADD M

DRC C

JNZ AGAIN

HLT

- Describe the following microprocessor addressing modes				(8 marks)

- Immediate

- Register indirect

- Direct

- Register