MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2016/2017

SECOND YEAR, FIRST SEMESTER EXAMINATION FOR THE DEGREE OF BACHELOR OF SCIENCE IN COMPUTER SCIENCE AND FORENSICS ,BACHELOR OF SCIENCE IN COMPUTER SCIENCE , BACHELOR OF SCIENCE IN INFORMATION TECHNOLOGYAND  BACHELOR OF SCIENCE IN COMPUTER TECHNOLOGY

SPS 3200:   BASIC ELECTRONICS

DATE: December, 2016						TIME: HOURS

INSTRUCTIONS: Answer questions one and any other two questions.

QUESTION ONE - (30 MARKS)

- (i)       Define the term PN junction.						(1 Mark)

- Draw a labeled diagram of a PN junction				(1 Mark)

- Give three applications of a PN junction				(3 Marks)

b)        The figure below shows an NPN transistor for which IΕ=10mA, D.C current gain

∝=0.967. Find  the values of IC and IB.					( 2 Marks)

c)         Define the following terms;

- Breakdown voltage							(2 Marks)

- Peak inverse voltage (PIV)						(2 Marks)

- Maximum power rating						(2 Marks)

d)        Define the term breakdown of an isulator.					(2 Marks)

e)        Using relevant diagrams distinguish between insulators, conductors and semiconductors.												(3 Marks)

f)       Explain the energy band theory.						(1 Mark)

g)      In a common base configuration, the value of collector current is 5.10mA and the value of emitter current is 5.18mA. Calculate the common base d.c current gain.	(2 Marks)

h)      Give an example of;

- Elemental semiconductor						(1 Mark)

- Compound semiconductor						(1 Mark)

- (i)  Name two types of currents in semiconductors.				(2 Marks)

(ii) Define doping.								(2 Marks)

j)       Define the following parameters as used in BJT transistors.

- βDC									(1 Mark)

- ∝DC									(2 Marks)

QUESTION TWO (20 MARKS)

- (i)   Define the term Fermi level.						(1 Mark)

(ii)  Distinguish between Fermi level in N-type semiconductor and Fermi level in P-type semiconductor with regards to change in temperature.			(4 Marks)

(iii)  Define the term intrinsic concentration.					(2 Marks)

- (i)    Determine the forward current for a germanium diode at room temperature when the   voltage across it is 0.3V. Assume room temperature is 27∘C and reverse saturation current is 1Na volt equivalent of temperature is r= T11,600			(4 Marks)

(ii)     Distinguish between common emmiter configuration and common collector configuration.								(4 Marks)

- (i)    Draw a circuit diagram of an NPN transistor 				(2 Marks)

(ii)   Define the term A.C current gain of a BJT transistor in common base configuration. (3 Marks)

QUESTION THREE (20 MARKS)

- Define the term OP-Amp.							(2 Marks)

- Design a circuit showing how an OP-Amp can be used as;

- Summer with three inputs						(3 Marks)

- Non-inverting amplifier						(3 Marks)

- Calculate the output voltages of an OP-Amp summer (Adder) for the following set of input voltages and Resistors.							(4 Marks)

V1=1 volt  		 V2=2 volt         V3=3 volt

R1=500 kΩ               R2=1MΩ          R3=1MΩ   &RF=1MΩ

- (i)   State three types of transistors.						(3 Marks)

(ii)  Differentiate between transfer and drain characteristics of a JFET.	(4 Marks)

- Name the material used in the construction of a tunnel diode.		(1 Mark)

QUESTION FOUR (20 MARKS)

- (i)   State the effect of the depletion region on a PN junction during reverse bias and forward bias.									(2 Marks)

(ii)  Give three applications of the diode whose symbol is shown below;

- With the aid of a diagram explain the working of a full-wave bridge rectifier.(4 Marks)

- A diode operating at 300∘k has V(forward) of 0.4V across it when the current through it is 10mA and 0.42V when the current is twice as large. What values of I0 and η allow the diode to be modeled by the diode equation

(6 Marks)

- Complete the diagram below of an NPN transistor by providing the associated currents and their directions.								(2 Marks)

- Differentiate between zener breakdown and avalanche breakdown.		(3 Marks)