JARAMOGI OGINGA ODINGA UNIVERSITY OF SCIENCE AND TECHNOLOGY

SCHOOL OF ENGINEERING AND TECHNOLOGY

UNIVERSITY EXAMINATIONS    FOR THE DIPLOMA IN BUILDING AND CIVIL ENGINEERING

2ND YEAR 2ND SEMESTER 2017/2018 ACADEMIC YEAR

CENTRE: MAIN CAMPUS

---------------------------------------------------------------------------------------------------------

COURSE CODE: TBC 2221

COURSE TITLE:    MECHANICS OF STRUCTURES II

EXAM VENUE:  LR 15                             STREAM: DIP IN BLD & CIV ENG

DATE:  21/12/2017                         	 EXAM SESSION: 9.00 – 10.30AM

DURATION: 1 ½  HOURS

--------------------------------------------------------------------------------------------------------

Instructions

- Answer question 1 (Compulsory) and ANY other two questions

- Candidates are advised not to write on question paper

- Candidates must hand in their answer booklets to the invigilator while in the examination room

QUESTION ONE (MARKS 30)

- Differentiate between bending stresses and direct stresses (4 marks)

- Concerning the bending of beams, define the following (6 marks)

- Radius of curvature

- Neutral axis

- Tension layer

- The diagram below shows part of a beam. The beam is subjected to traverse loads that cause the beam to bend. Using diagrams, show the stress strain and stress distribution across the beam section (5 marks)

- State two important observations that one has to make in using Macaulay’s method (4 marks)

- Briefly explain the following (6 marks)

- Second moment of area

- Polar moment of inertia

- Radius of gyration

- An aluminium plate is subjected to shear stress of 50N/mm2 and has a shear modulus of 26 kN/mm2 as shown below.  Determine (5 marks)

- The shear strain

- The total shear distortion,

QUESTION TWO (MARKS 20)

- The diagram below shows a section of a bent beam. With the aid of the diagram, show that  (simple bending formula)   (10 marks)

- State the assumptions made in coming up of the simple bending formula (5 marks)

- The steel beam carries a single concentrated load of 30kN at the midpoint and is simply supported. The beam has a span of 3m. Determine the maximum bending stresses and the curvature of the beam at the point of maximum bending moment. The Youngs modulus for steel is 200 x 103kN/mm2. (10 marks )

QUESTION THREE (MARKS 20)

- A cantilever beam is L m long and has a point load, W kN at the free end. The flexural stiffness is EI.

- Develop a mathematical expression for bending moment, M at a section x metres from the free end (2 marks)

- Develop an expression for slope,  (2 marks )

- Develop an expression for deflection, v  (( 2 marks )

- If L = 6m, W = 20kN and EI = 110 MNm2, determine the slope and deflection at the free end (4 marks)

- To test the shear strength of brickwork, a simple test was  devised using three bricks mortared together and loaded in a test rig as shown. If at some point in the test a load of 5kN is applied, determine the average shear stress along the surface between the mortar and the bricks.  (5.5 marks)

- The diagram below shows cross-sections of various beams. Arrange the beams in order of increasing moment of resistance (4.5 marks)

QUESTION FOUR (MARKS 20)

- Define and briefly explain the following terminologies as concerns beam deflection (10 marks)

- curvature

- slope

- deflection

- flexural stiffness

- slenderness ratio

- Using the diagram given below, develop expressions for

- Total compressive or tensile force on the section

- Moment of resistance on the section

(10 marks)

QUESTION FIVE (MARKS 20)

- A beam of symmetrical section of depth 200mm with a moment of Inertia 1 x 108 mm4 is simply supported over a span of 4m. What uniformly distributed load can it carry if the maximum bending stress is not to exceed 120 N/mm2? With the same permissible stress, what concentrated load may be carried by the beam at the midspan? (10 marks )

- Determine the second moment of area of the given rectangular cross-section about the neutral axis. (5 marks)

- A timber beam of solid rectangular cross-section is required to withstand an external bending moment of 11.25kN.m. If the maximum permissible bending stress is 5.0 N/mm2 and the beam has a breadth of 150mm (6 inch), calculate the minimum required depth, d (5 marks )