MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2016/2017

SECOND YEAR, FIRST SEMESTER EXAMINATION FOR THE DIPLOMA IN CIVIL ENGINEERING

ECV 2200: THEORY OF STRUCTURES II

DATE: DECEMBER, 2016                                                                          TIME: 11/2 HOURS

INSTRUCTIONS: -	Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- A truss of span 5m is loaded as shown below. Find;

- Reactions in the member of the truss					(7 Marks)

- Forces in the members of the truss using the method of joints	(8 Marks)

- A beam 6m long, simply supported at its ends is carrying a point load of 50kN at its centre.

The moment of inertia of the beam (EI) is given as equal to 78x 106mm4. If E for the material of the beam = 2.1 x 105N/mm2, calculate;

- Deflection at the centre of the beam					(4 Marks)

- Slope at the supports.							(4 Marks)

- A cantilever of length 2m carries a point load 20kN at the free end and another of 20kN at the centre. If 	E= 105N/mm2 and I = 108mm4 for the cantilever, then determine by moment of area method, the slope of the cantilever at the free end.		(7 Marks)

QUESTION TWO (15 MARKS)

A simply supported beam AB of length L and carrying a uniformly distributed load of w per unit length over the entire length is as shown below. Find;

- The slope at the ends								(7 Marks)

- The maximum deflection of the beam					(8 Marks)

QUESTION THREE (15 MARKS)

A simply supported bean of length 4m is subjected to a uniformly distributed load of 30kN/m over the whole span and deflects 15mm at the centre. Determine the crippling load if the beam is used as a column with the following conditions;

- One end fixed and another end hinged.					(8 Marks)

- Both ends pin jointed.								(7 Marks)

QUESTION FOUR (15 MARKS)

A cantilever 120mm and 200mm deep is 2.5m long. Determine;

- The reactions and the bending moments of the cantilever.			(7 Marks)

- The uniformly distributed load the beam can carry in order to produce a deflection of 5mm at the free end. Take E = 200 GN/ m2					(8 Marks)