W1-2-60-1-6

JOMO KENYATTA UNIVERSITY OF AGRICULTURE AND TECHNOLOGY

# UNIVERSITY EXAMINATIONS 2014/2015

END OF SEMESTER EXAMINATION FOR THE DEGREE OF BACHELOR OF RADIOGRAPHY

RAD 2202 : EQUIPMENT AND IMAGING

DATE:   DECEMBER    2014	                           	                TIME: 2  HOURS

INSTRUCTIONS:

ANSWER ALL  QUESTIONS  IN SECTION A AND B

ANSWER ONLY ONE QUESTION IN SECTION C

==========================================================

SECTION A: MULTIPLE CHOICE QUESTIONS  [20 MARKS]

QUESTION ONE

- The following are electromagnetic  radiations EXCEPT:

- White light

- X-rays

- Gamma rays

- Electron beam

- Bremsstrahlung :

- Is  used  to detect the amount  of radiation  leaving  the x-rays tube

- Is characteristics of the material used in the x-ray tube target

- Is energy possessed by an x-ray beam

- Is “breaking radiation” from the x-ray tube

- Tube current :

- Is current used to heat the filament of an x-ray tube

- Is current passing through the high tension circuit  of an x-ray tube

- Is the current that determines the amount of filtration  for the x-ray beam

- Is that  current passing  through the primary circuit for the  x-ray tube

- The  following are true  for  secondary radiation grids EXCEPT :

- The are used to minimize  scattered  radiation reaching  the  recording media

- They improve  the quality  of the radio graphic  image

- They assist in the immobilization of the  patient during  radiography examinations

- They are made of lead strips

- Geometrical unsharpness  is due to :

- Movement of patient during a radiographic exposure

- The dimension of the focal spot for the x-rays tube

- Grid movement during  a radiographic exposure

- Geometry of  the  x-ray beam

- X-ray tube rating:

- Is determined by the type of target material used for the X-ray tube

- Is highest  when the anode is stationary  during exposure

- Determines  the quality  of the radiographic  image

- Determines the turn around time of patients  for radiological  examinations

- The  penetrating ability of the X-ray beam:

- Determining  the self life of the x-ray tube

- Is determined by the  potential  differences  between the cathode  and anode of the x-ray tube

- Is not important  in digital radiography

- Is influenced  by the tube current

- One of the following is not true regarding the film-screen  combination:

- The film is exposed by light during exposure

- The image is revealed using a laser bean

- The film is placed between intensifying  screens in a light tight box

- Controlled illumination is required  for processing the film after exposure

- Mammography:

- Is a soft tissue  radiographic  examination of the whole body

- Utilizes  very low  beam energy

- Require  added filtration  on the x-ray beam

- Delivers a very high dose to the radiographer  during the procedure

- Beam collimation helps in:

- Lengthening the life of the x-ray equipment

- Reducing  the amount of scattered  radiation during  exposure

- Receding the exposure  time during  a radiographic  exposure

- None of the above

- The probability  of anger  electron production :.

- Increases with atomic number

- Decreases with the photon energy of the e-ray beam

- Decreases with the increase in atomic  mass

- Decreases with atomic number

- In a rotating  anode x-ray  tube

- The anode disc rotates during  a  radiographic  exposure

- The tube must be water  coated

- The anode is made of molybdenum

- The anode is stationary during a radiographic exposure

- The following statements are true concerning the space charge cloud around the cathode of an x-ray tube.  Which one id false?

- It inhibits the production  of electrons

- It is useful in improving the image  contrast for radiographic images

- The tube current is elevated with the use of  high potential  differences for the x-ray tube

- It is undesirable in radiography

- Anode  heat effect:

- Is due to pitting on the anode of the x-ray tube

- Is not  available in new x-ray tubes

- Is a disadvantage in radiography  and has no application

- Is not noticeable  on the radiographic image

- Which of the following is not a function of the tube shield /housing?

- Protects the x-ray tube from mechanical damage

- Holds  the oil for  the x-ray tube

- Holds the connections to the x-ray tube

- Help  in the generation of x-radiation

- Which of the following is not  a component  of the stationary  anode x-ray tube?

- Glass envelop

- Rotor  and its winding

- Tungsten Target

- Copper block

- The following statements are true  for fluoroscopy EXCEPT :

- It uses a crossed TV circuit

- It uses very low tube current

- Delivers high radiation  dose to the operator and the surroundings

- The tube is run at the highest speed possible

- Ultrasound  wave:

- Is electromagnetic in nature

- Propagates through longitudinal waves

- Is generated by fast moving particles

- Produces ionizing  effect  on the body tissues

- Digital imaging:

- Uses a special cassette  with an x-ray film

- Requires a processing laboratory

- Has low patient through-put

- Used a photostimulable  plate in its operation

- The following statements are true in reference to computerized tomographic  image. Which one is false?

- Acquisition involves measurement of x-ray transmission profiles through  a patient

- Image is acquired through a detector  arc

- Transmission profiles are used to view  the image

- The image  is composed of a matrix of picture  elements

SECTION B: SAQ’S  [30 MARKS]

- Explain  briefly the purpose of the following in medical imaging :-

- Secondary limiting  devices

- Beam centering devices

- Ultrasound transducers

- Image receptors

- Image intensifier tube   5x6=30 marks

SECTION  C: ESSAY QUESTIONS:

Answer only  ONE Question from this section – 20 marks

- Using a well labeled diagram  describe the construction features of a rotating anode x-ray tube  [20 marks]

- Describe the construction features of an ultrasound transducer  [20 marks]

- Digital imaging includes direct and computed radiography.  Discuss.  										 [20 marks]