MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2014/2015

SECOND YEAR SPECIAL/SUPPLEMENTARY EXAMINATION FOR DIPLOMA IN ELECTRICAL ENGINEERING

EEE 2204: ELECTRONICS II

DATE: OCTOBER 2015					   	             TIME: 11/2 HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- List three equipments that uses direct current for their/its operation.		(3 Marks)

- Draw waveform to distinguish regulated power supply from un-regulated.	(4 Marks)

- Define rectification.								(2 Marks)

- List two types of rectifier circuit.							(2 Marks)

- An alternating current supply of 240V is applied to a half wave single wave rectifier circuit through a transformer of turn ratio 7:1 shown below

Find direct current output voltage.							(4 Marks)

- Name the features of integrated circuit voltage regulators.			(4 Marks)

- Outline the difference between positive and negative feedback amplifier.	(4 Marks)

- Draw an operational amplifier symbol.						(2 Marks)

- Define the following terms:

- Slew rate.									(1 Mark)

- Common mode rejection ratio						(1 Mark)

- An Op-Amp has CMRR of 70dB 20,000.  Calculate its common mode gain.	(3 Marks)

QUESTION TWO (20 MARKS)

- List two applications of operational amplifier.					(2 Marks)

- Describe the operation of one of the listed application above of the operational amplifier (OP-AmP).										(5 Marks)

- With aid of circuit diagrams explain the connection of an operational amplifier to act as an amplifier.										(8 Marks)

QUESTION THREE (20 MARKS)

- The gain with negative feedback is reduced by a factor ____________ and if the loop gain is large compared to unity that is  the desired closed loop gain is given by __________												(2 Marks)

- Show that the closed loop gain with feedback is given by 		(6 Marks)

- With help of graphs show that bandwidth increase with negative feedback.	(4 Marks)

- An amplifier has open loop gain of 800 and a feedback of 0.01.  If the open loop gain changes by 20% due to temperature.  Find the percentage change in closed loop gain.													(3 Marks)

QUESTION FOUR (20 MARKS)

- Describe how voltage regulator can be converted into an adjustable regulator.	(3 Marks)

- The zener diode in the figure below had the following ratings:

- What would be the load voltage when the load current  varies from 10mA to 120MA?									(4 Marks)

- Also calculate voltage regulation of the regulator.				(2 Marks)

- With the help of a circuit diagram describe the operation of full wave rectifier with centre tapped transformer.									(5 Marks)