MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

SECOND YEAR FIRST SEMESTER EXAMINATION FOR THE DEGREE OF BACHELOR OF SCIENCE IN FOOD SCIENCE AND TECHNOLOGY

AFT 3200: FOOD ENGINEERING I

DATE: NOVEMBER 2015					   	                    TIME: 2 HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- Define or differentiate the following terms as used in food engineering:

- Density and specific gravity.

- Pasteurisation.

- Laminar flow.

- D-value.

- Dipolar rotation.								(5 Marks)

- Drying causes food to lose moisture.  A farmer used solar to dry 120kg of maize.  After drying he weighed the maize and got 80 kg.  A student took a sample (3g) of the solar dried maize and dried it further in an oven at 1050C for 24 hours.  The sample lost 30% of the weight.  What was the moisture content in wet and dry basis and the dry matter of the maize grains after solar drying by the farmer?						(5 Marks)

- A student had three samples, ice at 00C, water at 250 C and cooking fat at 250C.  How would their specific heat capacity and thermal conductivity compare?			(3 Marks)

- Differentiate between shear thinning and shear thickening.			(2 Marks)

- A rectangular stainless steel plate is used in manufacture of  a food heating vessel.  One side of the plate is 1000C and the other side is 250C.  Assuming steady-state conditions.  Calculate the rate of heat transfer through the plate.  The thermal conductivity of steel is 17W/(moC).									(3 Marks)

- A heat resistant spore was heat treated at 112 C.  At time zero, the number of survivors was 1million and after 8 minutes the number of survivors was 12,000.  Determine the D-value of the spore.										(3 Marks)

- Differentiate between freeze drying and spray drying giving an example of application of each process.									(4 Marks)

- Discuss importance of evaporation in food products.				(3 Marks)

- List two advantages of counter current heat exchanger.				(2 Marks)

QUESTION TWO (20 MARKS)

- Discuss two mechanisms through which a microwave oven heats food.		(10 Marks)

- Milk was pasteurized at 1200C for 96 seconds and obtained a 8 log reduction of the target bacteria.  The producer lowered the temperature to 100oC and obtained similar reduction of the bacteria after 8 minutes.  Determine the decimal reduction time and Z-value at the two temperatures.									(4 Marks)

- A 2cm thick steel pipe (thermal conductivity 43W/[moC]	 with 6 cm inside diameter is being used to convey steam from a boiler to process equipment for a distance of 40 m.  The inside pipe surface temperature is 115oC, and the outside pipe surface temperature is 90oC.  Calculate the total heat loss to the surroundings under steady-state conditions.(6 Marks)

QUESTION THREE (20 MARKS)

- State the law of conservation of mass.						(1 Marks)

- In preparation of ready to drink juice, streams of two concentrates were mixed.  Stream A contained 20% and stream B contained 60% (by weight) of sucrose.  How much of the two streams should be mixed to prepare 200 kg of juice with 30% sucrose concentration.												(5 Marks)

- Calculate the time necessary to dry a product from 90% to 25% moisture (wet basis) in an industrial dryer where 2 kg dry solid/m2 surface area exposed to the air is loaded.  It is given that the critical moisture content is 5 kg water/kg dry solid, the equilibrium moisture content is 0.033 kg water/kg dry solid and the drying rate at the critical moisture content is 3 kg water/m2h under the specified conditions.					(10 Marks)

- The following equipment are used in food processing.  Briefly highlight the principle they employ and their application:

- Rising film evaporator.

- Plate heat exchanger.							(4 Marks)

QUESTION FOUR (20 MARKS)

- Juice was pumped into a storage tank of 5M height and 3M diameter.  The inlet pipe was of 2.5 cm diameter and the mean flow rate velocity was 3m/s.  As a supervisor you wanted to fill the tank to 80% the capacity.  Determine how long it would take.  After it was filled to 80%, the inlet was closed and the outlet valve of 3 cm diameter was open and the juice flowed at 2m/s.  What was the level (height) of the juice in the tank after an hour?													(10 Marks)

- Differentiate between laminar and turbulent flow and discuss a simple experiment that you would carry out with fluid flowing through a pipe to demonstrate a laminar and turbulent flow.										(5 Marks)

- List two non-thermal food processing techniques highlighting the working principle.													(5 Marks)