MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

FIRST YEAR FIRST SEMESTER EXAMINATION FOR DIPLOMA IN ELECTRICAL ENGINEERING

EEE 2100: ELECTRCAL ENGINEERING PRINCIPLES I

DATE: NOVEMBER 2015					   	                 TIME: 11/2 HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- For the circuit shown in figure 1, use ohm’s law to determine;

- Current flowing in each resistor.

- Power dissipated in the 10 ohms resistor.					(8 Marks)

- Explain briefly the effect of temperature on the value of resistor for the following materials:

- Carbon									(11/2 Marks)

- Copper									(11/2 Marks)

- Define the following terms as used in magnetic circuits and state the unit of measurement in each case:

- Magnitomotive force.

- Magnetic flux density.

- Magnetic flux intensity.

- Permeability.								(6 Marks)

- With the aid of a clear diagram show the magnetic field formed when a dc current is passed through a solenoid.								(6 Marks)

- (i)  State Kirchhoff’s current law and voltage law.				(3 Marks)

(ii) Define the following terms as is used in electrostatics:

- Capacitance

- Permittivity								(4 Marks)

QUESTION TWO (15 MARKS)

- A wire of cross section area 1 mm2 has resistance of 200.  Find the resistance of a wire of the same length and material if its cross-sectional area is 1cm2.  		(5 Marks)

- With the aid of a labeled diagram explain briefly the constant current method of charging a battery.										(10 Marks)

QUESTION THREE (15 MARKS)

- State Faraday’s laws of electromagnetic induction.				(4 Marks)

- Use Kirchhoff’s laws to calculate the value of current flowing in each circuit in the circuit shown in figure 2.    								(11 marks)

QUESTION FOUR (15 MARKS)

- With the aid of a sketch show that the total capacitance, C, of three capacitors of capacitances C1, C2 and C3 are given by the equation;

(5 Marks)

- A magnetic circuit is existed by 120 turn coil.  The cross-sectional area and the length of the magnetic circuit are 5cm2 and 5 cm respectively.  When the current is 1.5 A the total flux is 0.3 milliwebber and when current is 5A the total flux is 0.6 milliwebber.  Find the

- Magnetic field strength.

- Relative permeability in each case.					(10 Marks)