MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

THIRD YEAR SECOND SEMESTER EXAMINATION FOR DIPLOMA IN CIVIL ENGINEERING

ECV 2350: CIVIL ENGINEERING QUANTITITES II

DATE: NOVEMBER 2015					   	                 TIME: 11/2 HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

A section plan of house 8.00 by 6.00 and a depth 900 mm by 750 mm as shown on the advanced drawing is partitioned at the centre with an internal wall 750 mm by 450 mm as shown on section R-B upto DPC level.  Prepare a taking off list of the substructure items to be used ranging from bush clearing to DPC. 							(30 Marks)

QUESTION TWO (15 MARKS)

- Abstract items in 1 above.

- Prepare a direct billing sheet of the above items.					(15 Marks)

QUESTION THREE (15 MARKS)

Using the data given below, build up a unit rate for mass concrete (1:2:6) in 450 mm bed per m2 in 10.00 by 6.00.

- Ballast Density		1600kg/m3

- Cement Density		1400kg/m3

- Sand Density		1600kg/m3

- 50 kg bag cement	Shs.1000.00

- Sand per tonne		Shs.2000.00

- Ballast per tonne	Shs.2000.00

- 200 litre capacity

mixer hire rate.	Shs. 500 per hour					(15 Marks)

QUESTION FOUR (15 MARKS)

Using neat sketches illustrate and define the following terms as used in staircase measurements with a simple drawing (104”) height building showing each.

Head room

Run

Rise

Width											(15 Marks)