MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

SECOND YEAR SECOND SEMESTER EXAMINATION FOR DIPLOMA IN ELECTRICAL ENGINEERING

EEE 2254: DIGITAL ELECTRONICS II

DATE: NOVEMBER 2015					   	                 TIME: 11/2 HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- Define the two demorgan’s theorems.						(4 Marks)

- List three types of logic families currently in existence.				(3 Marks)

- Outline the following:

- Flip-flop.									(2 Marks)

- Synchronous.								(2 Marks)

- Asynchronous.								(2 Marks)

- List three classes of registers using edge-triggered Dor J-K flip-flop.		(3 Marks)

- Simplify using Boolean expression

(4 Marks)

- Realize or implement the following Boolean expression using basic gates

- Y=AB+CD+EF								(2 Marks)

- ABC+DEF   using NAND gates only.					(2 Marks)

- Minimize the Boolean function using Karnaugh map

(4 Marks)

QUESTION TWO (15 MARKS)

- Briefly explain the characters of the following terms which constitute the logic families:

- Fan-in									(2 Marks)

- Noise immunity.								(2 Marks)

- Power dissipation 								(2 Marks)

- With aid of circuit diagrams explain briefly the difference in circuits operation of T.T.L and C MOS.										(6 Marks)

- State limitations of ripple counters.						(3 Marks)

QUESTION THREE (15 MARKS)

- Define multipleker.									(4 Marks)

- Digital logic circuits are implemented using gates.  The resultant circuits may posses their own symbol sketch a 4-line to 1-line multiplexer

- Using the gate implementation.

- Symbol.

- Truth-table 									(12 Marks)

QUESTION FOUR (15 MARKS)

- Compare briefly synchronous and asynchronous counters.			(4 Marks)

- Two warring factions A and B are to be reconciled by two friendly parties C and D.  Given that for negotiations to take place A and B and either C or D must be present.

- Draw the truth table that satisfies the given conditions.			(3 Marks)

- Draw the Karnaugh map from the truth table.				(2 Marks)

- Determine the Karnaugh minimum switching formula.			(2 Marks)

- Implement the switching formula (iv) above.				(2 Marks)

- Express the switching formula to NAND gates only.			(2 Marks)