MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

SECOND YEAR SECOND SEMESTER EXAMINATION FOR DIPLOMA IN ELECTRICAL ENGINEERING

EEE 2250: ELECTRICAL ENGINEERING PRINCIPLES IV

DATE: NOVEMBER 2015					   	                 TIME: 11/2 HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- With the aid of a phasor diagram explain the essential features in the representation of an unsymmetrical three phase system of voltages VR VY and VB by symmetrical components											(3 Marks)

- The following currents were recorded in the R, Y and B lines of a 3 phase system under abnormal conditions.

Calculate:

- Positive phase sequence components. 					(5 Marks)

- Negative phase sequence components.					(5 Marks)

- Zero phase sequence components.						(3 Marks)

- Return current in the neutral conductor.					(2 Marks)

- Define the term transients. 							(1 Mark)

- Define the term time constants as applied to transients.				(1 Mark)

- The voltage V=200 sin (314t+30o) is suddenly applied at t=0 to a circuit consisting of a 10 Ω resistor in series with a 0.1 H inductor.

- Deduce an equation showing how the current varies with time.		(7 Marks)

- Calculate the value of current 0.025 seconds after switching on.		(3 Marks)

QUESTION TWO (15 MARKS)

- State three conditions necessary for the production of transients.		(3 Marks)

- An indicator has a negligible resistance and an inductance of 250 mH and is conncected in series with a 500Ω resistor to a 24V, dc supply.

Determine:

- The time constant of the circuit.						(2 Marks)

- The steady stale value of the current flowing in the circuit.		(2 Marks)

- The voltage drop across the inductor at a time equal to two time constants.											(2 Marks)

- The voltage drop across the resistor after a time equal to three time constants.											(3 Marks)

- The expected time for the current to rise to 12% of its final value.	(3 Marks)

QUESTION THREE (15 MARKS)

- State and explain five types of transients. 						(5 Marks)

- A 20capacitor is connected in series with a 10kΩ resistor and the circuit is connected to a 40 dc supply.

Determine:

- The initial value of the current flowing.					(1 Mark)

- The time constant of the circuit.						(2 Marks)

- The value of the current one second after connection.			(2 Marks)

- The value of the capacitor voltage 0.4 seconds after connection.		(2 Marks)

- The time after connection when the resistor voltage is 6V.		(3 Marks)

QUESTION FOUR (15 MARKS)

Given that V0=10<102o, V1=13.5< -90o, V2=48<117o,

- Find the phase sequence components taking red phase as the reference.	(12 Marks)

- Sketch vector diagram representing the positive, negative and zero sequence components for the B phase.							(3 Marks)