MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

THIRD YEAR FIRST SEMESTER EXAMINATION FOR DIPLOMA IN CIVIL ENGINEERING

ECV 2303: ENGINEERING HYDRAULICS I

DATE: NOVEMBER 2015					   	                 TIME: 11/2 HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- Define the following terms:

- Open channel.

- Uniform flow.

- Gradually varied flow.

- Reservoir.

- Critical flow.								(5 Marks)

- Calculate the Hydraulic mean depth for a V section channel-made up of two 30o angles, when the depth of water at the centre is 225 mm.					(4 Marks)

- Briefly describe any three structural based type of dams.				(6 Marks)

- Sketch the following channel slopes associated with gradually varied flow.

- Mild slope.

- Steep slope.

- Critical slope.

- Adverse slope.								(6 Marks)

- List the advantages of hydro-electrical power plant.				(5 Marks)

- List the main differences between a reaction and an impulse turbine.		(4 Marks)

QUESTION TWO (15 MARKS)

- With the aid of a drawing describe the main storage levels of a dam.		(8 Marks)

- Describe with a drawing, the working of a pelton wheel hydroelectric power plant.											(7 Marks)

QUESTION THREE (15 MARKS)

- Show that for a circular culvert of a diameter D velocity of flow will be a maximum when the depth of flow h at the centre is approximately 0.81D.  Use the chezy formula.	(9 Marks)

- A sewer diameter D=0.6m, has a slope of 1 in 200.  What will be the maximum velocity of flow that can occur, and what is the discharge at this velocity?  Take C=55SI units.												(6 Marks)

QUESTION FOUR (15 MARKS)

- Describe the working of a Kaplan Turbine.					(9 Marks)

- Show that for a rectangular channel of depth D and width B, the proportions which will make the discharge a maximum for a given cross-sectional area A are given by A=2D2.

(6 Marks)