MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

SECOND YEAR SECOND SEMESTER EXAMINATION FOR DIPLOMA IN ELECTRICAL ENGINEERING

EEE 2255: POWER ELECTRONICS

DATE: NOVEMBER 2015					   	                 TIME: 11/2 HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- Figure 1 shows an initially relaxed circuit.  Thyristor T is turned on at t=0 seconds.

Given:	L=5 millihenrys.

C= 20 microfarads

Vs=200 volts

Calculate:

- Conduction time and thyristor.

- Voltage across thyrisor.

- Capacitance after SCR is turned off.					(7 Marks)

Figure 1

- Figure 2 is structure of the thyrisor device.

Figure 2

- Name the device.

- Explain its operation.

- Construct its symbol.							(6 Marks)

- (i) Explain the various modes of operation of a thyristor.

(ii) Discuss conditions which must be satisfied to turn on a thyrisor using gate circuit.												(10 Marks)

- (i) What is power electronics?							(1 Mark)

(ii) List 2 advantages of power electronics.					(2 Marks)

(iii) Figure 3 shows how forward breakover voltage with gate current.  Explain the relationship shown in this diagram.						(4 Marks)

Figure 3

QUESTION TWO (15 MARKS)

- A two transistor anology of a SCR has the following:

PNP transistor gain = 0.42

NPN transistor gain = 0.54

Rated gate triggering current = 68 mA

- Derive the expression for the anode current.

- Calculate the value of anode current.						(4 Marks)

- Explain:

- Forward voltage triggering.						(1 Mark)

- Light triggering.								(1 Mark)

- Temperature triggering.							(1 Mark)

- Define:

- Holding current.								(1 Mark)

- Latching current.								(1 Mark)

- Figure 4 is a single phase half wave converter.

Figure 4

Construct wave forms of:

- Vs or input signal.								(1 Mark)

- Firing signal 1g.								(1 Mark)

- Load current io.								(2 Marks)

- Voltage across thyristor (VT)							(2 Marks)

Assuming firing angle of

QUESTION THREE (15 MARKS)

- Describe RC filters listed indicating ideal and practical responcer to input signals:

- Bound stop.									(2 Marks)

- Low pass.									(2 Marks)

- Band stop.									(2 Marks)

- High pass.									(2 Marks)

- A low pass filter consists of an inductance of 5000 PF.  Calculate its:

- Source resistance.								(1 Mark)

- Load resistance.								(1 Mark)

- Cut-off frequency.								(1 Mark)

- Explain a thyristor.

- Hard firing.									(1 Mark)

- Forced commutation.							(1 Mark)

- Line commutation.								(2 Marks)

QUESTION FOUR (15 MARKS)

- You are provided with the following devices:

- SW (Switch)

- L (inductor)

- R (load)

- FD (free wheeling diode)

- VS (source of power)

- Sketch a chopper from above information.					(1 Mark)

- State a device used instead of SW.						(1 Mark)

- Explain the principle of operation of the sketched device.			(1 Mark)

- Draw the expected input and output voltage and current wave for ms.	(1 Mark)

- If the input voltage of the device constructed is 50v and duty cycle is 5 milliseconds, calculate output voltage.							(3 Marks)

- Explain the following DC choppers:

- Step up.									(11/2 Marks)

- Step down.									(11/2 Marks)

- Discuss the operation of D in the following circuit figure 5.			(5 Marks)

Figure 5