MERU UNIVERSITY OF SCIENCE AND TECHNOLOGY

P.O. Box 972-60200 – Meru-Kenya.

Tel: 020-2069349, 061-2309217. 064-30320 Cell phone: +254 712524293, +254 789151411

Fax: 064-30321

Website: www.must.ac.ke  Email: info@must.ac.ke

University Examinations 2015/2016

FIRST YEAR, SECOND SEMESTER EXAMINATION FOR THE DEGREE OF BACHELOR OF SCIENCE IN COMPUTER TECHNOLOGY

CIC 3175: PRINCIPLES OF ELECTRICAL ENGINEERING

DATE: NOVEMBER 2015                                                                                     TIME: 2HOURS

INSTRUCTIONS: Answer question one and any other two questions

QUESTION ONE (30 MARKS)

- State Kirchaff laws										(4 Marks)

- Define the following terms:

- Passive elements									(2 Marks)

- Active elements									(2 Marks)

- Four resistors 3Ω, 4Ω, 5Ω and 6Ω are connected in series. If the total current is 1.0A. Find the voltage across each resistor.								(5 Marks)

- Differentiate between short and open circuit						(4 Marks)

- A coil consists of 100 turns of copper wire and has a cross sectional area of 0.8mm2. The mean length per turn is 80cm and the resistivity of copper is 0.02µ Ωm. Find the resistance of the coil.												(5 Marks)

- Three capacitors 3 µf, 6mf and 12 µf respectively are connected in series to a 220V d.c supply. Find:

- Total capacitance								 (2 Marks)

- Charge in each capacitor							(3 Marks)

- State three essentials features of indicating analogue instruments				 (3 Marks)

QUESTION TWO (20 MARKS)

- Define the following terms:

i) Inductance

ii) Conductance									(2 Marks)

- Using node voltage method, find the current in the 3 Ω. resistor of the network shown in figure 1		 										(8 Marks)

- A coil having an inductance of 0.5H has its current reduced from 5A to 2A in 0.5s. Calculate the mean value of the e.m.f induced in the coil. 					(5 Marks)

- What are the causes of transit disturbances					 (3 marks)

QUESTION THREE (20 MARKS)

- state the maximum power transfer theorem						(3 Marks)

- show that the power transfer for the source to load is maximum when the resistance R of the load is equal to the internal resistance (r) of the source 						(7 Marks)

- A moving coil instrument gives a full-scale deflection of 16mA and has a resistance of 4Ω. calculate the resistance required:

    - in parallel to enable the instrument to read up to  1A					 (6 Marks)

    - in series to enable it read up to 10V 							(4 Marks)

QUESTION 4 (20 MARKS)

      - Using kirchoff’s network calculate currents through the 18 Ω. resistor in the circuit below													(8 Marks)

      - With the aid of circuit diagram differentiate between forward and reverse based diode. (8 Marks)

      - State the assumptions made when calculating steady current in electrical circuits.	 (4 Marks)