W1-2-60-1-6

JOMO KENYATTA UNIVERSITY

OF

AGRICULTURE AND TECHNOLOGY

UNIVERSITY EXAMINATIONS 2015/2016

SECOND YEAR FIRST  SEMESTER

EXAMINATION FOR THE DEGREE OF BACHELOR OF SCIENCE AGRICULTURAL AND BIOSYSTEMS ENGINEERING

SPH 0102: PHYSICS

DATE:   APRIL, 2016						     TIME: 2 HOURS

INSTRUCTIONS:	ANSWER ALL QUESTIONS AND ANY TWO OTHER  QUESTIONS

QUESTION ONE (20 MARKS)

- Distinguish between the following terms:

i)		Distance and displacement						[2 marks]

ii)		Velocity and speed							[2 marks]

- State the THREE Newton’s law of Linear motion			[1 marks]

- i)	Define viscosity							[1 marks]

ii)	Oil pour out more slowly than water from a container.

Explain this phenomenon.						[1 marks]

iii)	State two ways of increasing surface tension force.		[2 marks]

iv)	A steel ball bearing is allowed to fall freely in a viscous liquid.  State

the a condition necessary for it to attain terminal velocity. 	[1 marks]

d)	Explain the following phenomenon as used in waves.

i)	Interference							[1 marks]

ii)	Distraction							[1 marks]

iii)	Doppler’s effect						[1 marks]

e)	A person standing 150m from tall cliff claps his hands and hears an echo

0.4 seconds later.  Calculate the velocity of sound in air.		[2 marks]

f)	Differentiate between alternating and direct currents.		[1 marks]

g)	List and TWO optical instruments.					[1 marks]

QUESTION TWO (20 MARKS)

a)	Sate the following laws.

i)	Kirchhoff’s voltage law.	 					[1 marks]

ii)	Kirchhoff’s current law.						[1 marks]

iii)	Ohm’s law								[1 marks]

iv)	Coulomb’s law of electrical charges				[1 marks]

b)	A copper wire has a length of 200m and a cross-section area of 2mm2.

Find its resistance (take resistivity of copper as equal to

1-7 x 10-8Ω m)								[1 marks]

e)	Calculate the energy stored by the capacitor charged to 500 microfaroids

the potential difference across its plate is 10 volts.			[4 marks]

d)	Find the capacitance of a parallel-plate capacitor of plate area 0.01m2

with an air dielectric  of thickness 0.01 meters. (E0=8.85 x 10-12 F/M)

[3 marks]

e)	A model car moves on a circular track of radius 0.3m at 2 revolutions per

seconds.  Calculate its:-

i)	Frequency (f)							[1 marks]

ii)	Angular speed (w)							[2 marks]

iii)	Linear speed (v) of the car					[2 marks]

QUESTION THREE (20 MARKS)

a)	A stone is thrown vertically upwards with an initial velocity of 30mls

neglecting air resistance, calculate:

i)	The maximum height reached.					[3 marks]

ii)	The maximum time taken before it reaches the ground.	[3 marks]

iii)	Take acceleration due to gravity.			        [10 marks]

b)	Suppose a train with velocity ∪	accelerates with a uniform acceleration	for a time  t  and reaches a velocity  V.  Show that the distance s travelled

by the train in the time  t is given by s= ∪t + ½ a+2		        [10 marks]

c)	i)	State Hooke’s law.							[1 marks]

ii)	Define the following terms as used in elasticity.

(I)	Tensile strain						[1 marks]

(II)	Tensile stress						[1 marks]

(III)	Young’s Modules (E)					[1 marks]

iii)	8kg mass is attached to the end of a vertical wire of length 3m

diameter 0.68MM and extension 0.32mm.  Determine the wire’s

(I)	Tensile strain						[2 marks]

(II)	Tensile stress						[2 marks]

(III)	Young’s modulus (E)					[2 marks]

QUESTION FOUR (20 MARKS)

a)	i)		State faraday’s and lens’s law of electromagnetic induction.

[2 marks]

ii)	Outline three applications of electromagnetic induction.	[3 marks]

b)	An alternating voltage is given by following expressive;

V= 10 sin 10  ᴧ t.  Calculate:-

i)		Its maximum valve						[1 marks]

ii)	Its frequency							[2 marks]

iii)	Its period								[2 marks]

c)	A transformer inputs a potential difference of 80kv to an x-ray tube from a

240v a-c mains supply.  A current of 10MA flows in the x-ray tube.  If the

transformer is 100% efficient, calculate:-

i)		Ratio of number of turns of secondary to number of turns of the

primary.								[2 marks]

ii)	Current in primary coil						[2 marks]

iii	State, giving reasons which of the coils of the transformer is thicket

[2 marks]

d)	A house has 10 rooms each with one 240v  80w bulb.  If the bulbs are

switched on from 7.00pm to 10.00pm.

i)		Calculate the electrical energy consumed per day in kilowatts hour.

[2 marks]

ii)	Find the cost per week for lighting these rooms at ksh. 7.80 per unit

[2 marks]

QUESTION FIVE (20 MARKS)

a)	State the laws of:-

i)	Refraction								[2 marks]

ii)	Reflection								[2 marks]

b)	Distinguish between a real image and a virtual image.		[2 marks]

c)	By use of a ray diagram, describe the image formed by convex mirrors.

[4 marks]

d)	State Two factors that may reduce the stability of a vehicle while going

through a banked bend.							[2 marks]

e)	i)	Define critical angle.						[1 marks]

ii)	Name two conditions necessary for total internal reflection to

occur.									[2 marks]

f)	i)	An object is placed 12cm from a converging lens of focal length

18cm.  Find the position of the image formed.		[3 marks]

ii)	Name two characteristics of an image formed by a convex lens.												[2 marks]