W1-2-60-1-6

JOMO KENYATTA UNIVERSITY

OF

AGRICULTURE AND TECHNOLOGY

UNIVERSITY EXAMINATIONS 2017/2018

YEAR II SEMESTER I EXAMINATION FOR THE DEGREE OF BACHELOR OF ARCHITECTURAL TECHNOLOGY

ABA 2201: BUILDING SERVICES

DATE:   JANUARY 2018	         	              TIME: 2 HOURS

INSTRUCTIONS:  ANSWER QUESTION ONE AND ANY OTHER TWO QUESTIONS

QUESTION ONE (30 MARKS)

- Comment why it is necessary in the planning stage of a project to liaise with utility service providers like water boards, power and lighting companies urban councils in the initial application made by a developer for planning.	[7 marks]

- What is the purpose and objective of service ducts in high rise buildings?  Describe the following types of service ducts;

- Walk-in-vertical duct.

- Built out vertical duct	[11 marks]

- What determines the waste handling system to be used in any particular situation?  Comment on Hospital waste disposal systems.		[8 marks]

- Outline the principle of hot and cold water supply systems in buildings.	[4 marks]

QUESTION TWO (20 MARKS)

- How does an indirect cold water supply system in buildings work?  State any disadvantages of the system.	[8 marks]

- Briefly explain the following systems of water supply to the buildings;

- Constant system

- Intermittent system

- Private water supply system.	[9 marks]

- What is the principle requirements of a storage tank for drinking water?  Illustrate by a simple line diagram.	[3 marks]

QUESTION THREE (20 MARKS)

- Describe a sanitary appliance, its functions and general requirements.	[9 marks]

- Comment on pipe connections requirements to sanitary appliances and traps in building installations.	[5 marks]

- Sketch and describe the following sanitary appliances;

- Slop hopper

- Washing fountain	[6 marks]

QUESTION FOUR (20 MARKS)

- Outline the following systems of drainage and comment on their preferences;

- Combined drainage system

- Partially separate system	[8 marks]

- What factors are likely to determine a layout for a small domestic drainage scheme?		[2 marks]

- What is FIRE PRECAUTION in buildings?  Explain the term FIRE GRADING as used in fire resistance of building elements or components and illustrate with well annotated sketches the following;

- One-Hour five check flush door with frame.

- One-Hour fire resistance concrete beam.

[10 marks]

QUESTION FIVE (20 MARKS)

- Using a simple line diagram describe the sequence of electrical power supply control.		[4 marks]

- Differentiate ventilation from Air Conditioning in Building Installations.	[6 marks]

- Briefly specify how the following details of a building design affects natural indoor ventilations;

- Geometrical configuration of the Building envelope

- Location of openings

- Total area openings in the pressure and sanction zone.	[6 marks]

- What are the three major Architectural considerations that require integration when planning for elevators?				[4 marks]